2.0Plán projektuBSRO.PS8066:220.1STATUS - Sběr,zpracování a uchování údajů z komerčních bankPlán projektu ã 1997 Logica s.r.o. 00Project Plan Cortex Output Description template Version 1.0.2210.2.1997

Information System

User Manual

External interface CS OTE

BINARY API message format of electricity Intraday market

This document and its content are confidential. The document must not be reproduced in whole or in part or disclosed to third parties or used for purposes other than those for which it was provided without prior written approval from company OTE, a.s.

Content

List of figures	5

List of tables	6

History of changes	7

Reference documents	8

Abbreviations	9

1.	Introduction	10

2.	External interface description	11

2.1.	Communication protocol	11

2.2.	Connecting to MQ server	11

2.3.	Message exchange types	12

2.3.1.	Request-Response communication	12

2.3.2.	Mass messages – Broadcast	13

2.3.3.	Distribution rules	13

2.3.4.	Sequence use for Broadcast messages	14

2.4.	Invalid and unrouteable requests	15

2.5.	Failover processing	16

2.6.	General information about communication messages	16

2.6.1.	AMQP attributes	16

2.6.2.	Protobuf convention	16

2.6.3.	Quantity values within messages	17

2.6.4.	Price values within messages	17

2.6.5.	Format of date items within messages	17

2.6.6.	Heartbeat message	17

2.6.7.	Standard message header	18

2.6.8.	Individual message parameter descriptions	18

2.7.	Communication scenarios	18

2.7.1.	User log-in, log-out	18

2.7.2.	Base scenario of bid submission – error processing	19

2.7.3.	Bid manipulation	20

2.7.4.	Trade callback	22

2.7.5.	Trade cancellation	23

2.7.6.	Public bid data request	24

2.7.7.	Public trade data request	25

2.7.8.	Information message request	25

2.7.9.	Product and market contract request	26

2.7.10.	Market status request	27

2.7.11.	Capacity data request	27

2.7.12.	Market area request	28

2.7.13.	Delivery area request	28

2.7.14.	Market state distributed message	28

2.8.	Communication messages	29

2.8.1.	General requests and responses	30

2.8.2.	Bid submission and management	32

2.8.3.	IM Trade management	40

2.8.4.	Market information	40

2.8.5.	IM reference data	52

2.9.	Scenarios for the current automatic communication through the KSP/KSM communication server	54

2.9.1.	Configuration/modification/response to the new IM limit	54

2.9.2.	Message indicating the transfer of a portion of the IM limit into the main trading limit	55

3.	Using the electronic signature	57

# List of figures

Figure 1 - Communication with MQ server	11

Figure 2 - Connection to MQ server and message flow architecture	12

Figure 3 - User login/logout sequence diagram	19

Figure 4 - General request sequence diagram	19

Figure 5 - Order submission and trade establishment sequence diagram	20

Figure 6 - Order modification and trade establishment sequence diagram	21

Figure 7 - Bulk order modification (or deactivation) and the subsequent order request sequence diagram	22

Figure 8 - Trade recall sequence diagram	23

Figure 9 - Trade cancellation sequence diagram	24

Figure 10 - Order request sequence diagram	24

Figure 11 - Trade request sequence diagram	25

Figure 12 - Market messages request sequence diagram	26

Figure 13 - Product and contract request sequence diagram	26

Figure 14 - Market state request sequence diagram	27

Figure 15 - H2H matrix request sequence diagram	27

Figure 16 - Market area request sequence diagram	28

Figure 17 - Delivery area request sequence diagram	28

Figure 18 - Market state distributed message sequence diagram	29

Figure 19 - SignedMessage creation	57

Figure 20 - Digital signature message verification with original message extraction	57

# List of tables

Table 1 - Distribution rules overview	14

Table 2 -SequenceNumbersRprt message structure	15

Table 3 - Message attributes according to AMQP	16

Table 4 - Message header	18

Table 5 - Login request message structure	30

Table 6 - User report message structure	31

Table 7 - Logout request message structure	31

Table 8 - Logout report message structure	32

Table 9 -  Acknowledgement response message structure	32

Table 10 - Error response message structure	32

Table 11 - Add order request message structure	34

Table 12 - Order modify request structure	36

Table 13 - Order request message structure	36

Table 14 - Order execution report message structure	39

Table 15 - Modify all orders request message structure	40

Table 16 - Trade recall request message structure	40

Table 17 - Public order books request message structure	41

Table 18 - Public order books report message structure	42

Table 19 - Message request message structure	42

Table 20 - Message report message structure	43

Table 21 - Trade capture request message structure	44

Table 22 - Trade capture report message structure	45

Table 23 - Public trade confirmation request message structure	46

Table 24 - Public trade confirmation report message structure	47

Table 25 - Contract information request message structure	47

Table 26 - Contract information report message structure	48

Table 27 - Product information request message structure	49

Table 28 - Product information report message structure	50

Table 29 - Market state request message structure	50

Table 30 - Market state report message structure	51

Table 31 - Hub-to-Hub matrix request message structure	51

Table 32 - Hub-to-Hub matrix report message structure	52

Table 33 - Hub-to-Hub matrix report message structure	52

Table 34 - Delivery are information request message structure	52

Table 35 - Delivery area information report message structure	53

Table 36 - Market area information request message structure	53

Table 38 - Market area information report message structure	54

Table 39 - Response reason codes with msg code 483	55

Table 40 - SignedMessage message structure	57

# History of changes

| Date | Version | Change description |
| --- | --- | --- |
| 12.12.2025 | A | Document creation. |
| 16.03.2026 | B | Changes in accordance with .proto definition |
| 14.04.2026 | C | Chapter: Mass message for sequence numbers reconciliation |

# Reference documents

- OTE-COM test environment access configuration manual

- OTE-COM production environment access configuration manual

- Změna_formátu_zpráv_OTE-COM_ELE_protobuf_vs_XML

- .PROTO definition

# Abbreviations

| Zkratka | Popis |
| --- | --- |
| BIN API | Binary API |
| CS OTE | Central System of Mmarket Operator (OTE) |
| EAN | European Article Number – unambiguous international generic identificator |
| EIC | Energy Identification Code – unambiguous international energetics identificator |
| FS | Financial security |
| IM | Continuous Intraday Market with Electricity |
| IS OTE | Information system of market operator |
| MP | Market Participant |
| Protobuf | Protocol buffers – Language and platform neutral spreadable mechanism of serialization of structured data from Google (https://protobuf.dev) |
| SFS | System of Finance Security (OTE) |
| XML | Extensible Markup Language |

# Introduction

The aim of this document is to provide a specification for electricity continuous intraday market with electricity interface including AMQP server and the usage of BINARY API protocol buffers message content format.

If external participants use OTE client´s application, then it already contains this interface and communication. In case external participants request connection of new OTE IM to their systems, then this document should provide description of necessary changes in the interface for implementation BINARY API protocol buffers message content format.

# External interface description

By reason of ensuring of high throughput and quick distribution of messages from the IM markets, CS OTE expands by another platform supporting the AMQP protocol. At these markets automatic communication will be only performed through communication with the AMQP RabbitMQ server. In comparison with the current automatic communication solution a special setup/permission will not be required by OTE. The interface for AMQP RabbitMQ server will be available to all participants without client identification (identification through certificate).

Participant has to perform implementation of his client which will connect to the MQ server. Participant will use his client for sending of his requests and receiving responses and mass messages. It is possible to use the AMQP client library RabbitMQ – see web site of the product www.rabbitmq.com.

Process of establishing of communication and individual communication scripts are described in the following parts.

## Communication protocol

Communication with the MQ server is being processed via the AMQP (Advanced Message Queuing Protocol) protocol. It is a public standard for communication layer of applications based on data exchange through messages. Implementation will be performed via the MQ server RabbitMQ version 4.0.3.

AMQP standard defines basic entities:

- Exchange – input point for message receipt

- Routes – routing (distribution) of message

- Queue – output queue of messages

Figure 1 - Communication with MQ server

## Connecting to MQ server

For the connection, an external participant needs to know the following technical information: RabbitMQ server address, port and virtual host identification - this information is specified separately for various OTECOM application environments in the documents "Instruction for the first access to the production/test environment of OTE-COM application" (see http://www.ote-cr.cz/documentation/electricitydocumentation/market-documentation). Description of how to connect a custom client application is available at http://www.rabbitmq.com/api-guide.html.

The first step is to establish connection „connection“ to MQ server. Client´s certificate is necessary for creation of „connection“. This certificate has to be registered in OTE systems first.

These channels connect to the individual „queue“ which serve for mutual communication between client and server.

To establish a connection, the external participant must have the following technical details: RabbitMQ server address, port and virtual host identifier – these details are specified individually for each OTE-COM application environment in documents  [1], [2]. The description of connection method from a custom client application is available at the following address http://www.rabbitmq.com/api-guide.html. External participant provides his own client certificate to OTE.

Figure 2 - Connection to MQ server and message flow architecture

## Message exchange types

There are two basic types of communication used for Client – MQ server communication:

- Request-response – requests initiated by client on which the MQ server will asynchronously respond. The response is sent only to initiator of the communication

- Mass message (broadcast) – message distribution from the MQ server to clients. Distribution is performed on the basis of defined distributional rules and access rights.

### Request-Response communication

Each user has its own private “Exchange” named “market.exchanges.clientRequest.[USER_ID]“ created in the RabbitMQ server, which is used for sending requests from client to the MQ server. Only the given user has permission to write to the specific exchange.

The user uses the queue named “response queue” for receiving messages addressed to the user, which is not pre-created in AMQP server, but is created by each client individually. The client must create their own anonymous queue during the application start with automatically generated name, whose name is then used in element “reply-to” in all messages.

The queue must be created with the following technical details: durable=false, autoDelete=true, exclusive=true.

Types of reqeuests:

- Instruction (Management request) – bid creation, modification, anullation

- Request (Inquiry request) – request for trading data

The response is immediately returned to the user (distributed to ResponseQueue) via message Acknowledgement Response (AckResp) during the“Management request” operation. The response to the submitted request is sent (distributed into BroadcastQueue) after the system processes the request. If the request modifies the business data, a broadcast message with relevant content is sent to all users for whom the change is relevant.

The relevant response is sent to the user’s private queue (ResponseQueue) during the “Inquiry request” operation.

Request type “Management request” is secured by an electronic signature, wrapped in a SignedMessage structure (see chapter. 3 Using the electronic signature) to ensure integrity and non-repudiation. It is generally required to populate the type (see chapter 2.6.1 AMQP attributes) attribute during the client-IM application communication.

### Mass messages – Broadcast

The system provides two basic types of mass messages:

- Market data messages – messages that contain changes to business data and market state distributed to all logged-in users who have the appropriate rights for the relevant markets.

- Heartbeat messages – messages used to verify active client connection

Every user is connected to their own private queue, created in RabbitMQ and named “market.broadcastQueue.[USER_ID]“ to receive messages. If the user does not consume messages regularly, their queue might become full and new messages will not be included in their queue. This poses a risk, that they will not receive all the information from the market.

### Distribution rules

Description of distribution rules are described in the following table. Some keys are dynamically defined based on the actual market configuration and user access rights.

| Routing key | Description |
| --- | --- |
| public | Public information, distributed to all users |
| public.<market_id> | Public information about the current market, distributed to all users, who have access to that market |
| public.trade.<product_name> | Public information about trades, distributed to all users, who have access to the relevant product |
| PRTC_<partic_id> | Relevant current market participant information |
| <product_name> | Relevant product information |
| <product_name>.<delivery_area> | Relevant product and distribution area information |
| <product_name>. PRTC_<partic_id> | Relevant PARTIC_ID information based on product |
| trade | Trade information for administrators only (consists of both sides of the trade) |
| halfTrade.<product_name>. PRTC_<partic_id> | Private information about established trades (contains only the participant’s side of trade) |
| USR_<user_id> | Private messages addressed exclusively to the user |

Table 1 - Distribution rules overview

There is a user-specific example provided as a showcase:

User: 123, Participant: 12, Market access: INTRADAY, Available products: INTRADAY_1H, Available areas: CZ

User will receive messages, that will be sent with some of the following Routing keys:

- public

- public.INTRADAY

- public.trade.INTRADAY_1H

- PRTC_12

- INTRADAY_1H

- INTRADAY_1H.CZ

- INTRADAY_1H.PRTC_12

- halfTrade.INTRADAY_1H.PRTC_12

- USR_123

### Sequence use for Broadcast messages

Sequence number is used to identify the order of mass messages, for the purpose of detecting whether any message has not been lost. Sequence number is not contained directly in the message payload, but it is included in the AMQP message header as an attribute called “market-group-sequence”.

Sequence is always incremented by one for each mass message. Sequences are kept only in memory (they are not stored), which means they will be set to 0 during a server restart or termination. If client receives unexpected value (different value from the last number + 1), client should request current market data from CS OTE system.

Sequence numbers are calculated individually for Routing keys (routing keys – attribute “market-group-id” in message header). Each distribution list has its own sequence numbers. Queues connected to the default mass message exchange with the same Routing key will receive duplicate sequence number.

#### Mass message for sequence numbers reconciliation

It is a OTE-COM’s feature ensuring more robust communication with MP. There is distributed a special mass message to each connected client containing information about last used sequence numbers for each Routing key of public distribution messages.

SequenceNumbersRprt distribution messages must follow these rules:

- One and the exact same distribution message is sent to all clients

- Each client receives the distribution message, which contains the last used sequence numbers for all public Routing keys, including those that are not relevant

- Every client selects its relevant Routing keys

- Each Routing key is published only once

- A message is distributed every x seconds

  - The sequence number of a Routing key is the last known value of the exact moment of the creation of the SequenceNumbersRprt message

  - Every time the server restarts, the sequence number is reset to null

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| SequenceNumbersRprt | MSG |  |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| seq_numbers | FIELD | m | 1..n | Structure |  |
| routing_key | FIELD | m |  | String | Routing Key, Each Routing Key is present only once |
| sequence | FIELD | m |  | Integer(64) | Latest sequence number of the Routing Key |

Table 2 -SequenceNumbersRprt message structure

## Invalid and unrouteable requests

It is important that clients create a valid protobuf (proto3) message content, which they send to CS OTE. Messages with protobuf (proto3) content that CS OTE system can not read will be rejected.

If the CS OTE system is unable to process a request because it is invalid or can not be processed, a negative response will be sent. The response contains details including the reason why the request was not processed.

If the CS OTE system can not process a request due to an invalid or missing message header version, a native error response is sent. In this case, the response has an attribute content-type set to value market/error. The message body contains an error message encoded in UTF-8. Discovery of a message validation error within the CS OTE system leads to sending a native error response. These validation errors occur in the following cases:

- Missing AMQP message attribute user-id

- Missing AMQP message attribute content-type

- Missing AMQP message attribute reply-to

- Missing AMQP message attribute correlation-id

- Missing AMQP message attribute type

- Unknown AMQP protobuf (proto3) message content

If the CS OTE system can not process a request because it is not running (due to an outage or during a restart), the request will be cancelled on the AMQP server side and the client will be informed via their “return listener”.

## Failover processing

In case the AMQP is not running (due to an outage or restart), the client connection is lost. If the client has a registered a “shutdown listener”, they will receive an outage notification from the AMQP server. After successfully reconnecting to the AMQP server, the client must log in again.

## General information about communication messages

### AMQP attributes

The AMQP attributes used for communication between the client and an IM application.

| AMQP Message Atribut | Popis |
| --- | --- |
| content-type | Contains information about the used payload version as well as the used message type. Valid content-type definitions are (version number has to be filled with the used version): - market/request; version=x (Used by the client when sending requests) - market/response; version=x - market/broadcast; version=x - market/heartbeat; version=x - market/error; version=x Current version of messages is 5. |
| type | Contains fully qualified name of the message |
| reply-to | Contains the queue name a response has to be sent to |
| user-id | Contains the login-id of the logged in system |
| correlation-id | Contains the request message id generated by client |
| expiration | Contains an optional entry specifying if the request should be deleted if not executed within the specified time |
| contentEncoding | Contains gzip, if messages are compressed (content is encrypted using gzip method); property is null if messages are not compressed. Message compressing can be activated per message type (e.g. OrdrExecutionRprt) . |
| market-group-sequence | Identify the order of the broadcasts counted for „market-group-id“. Only for broadcast message. |
| market-group-id | Identification of routing key belongs to attribute „market-group-sequence“. Only for broadcast message. |
| timestamp | Timestamp of distributed message fulfilled by RabbitMQ server. For more information you can see at https://www.rabbitmq.com/releases/rabbitmq-java-client/v3.6.1/rabbitmq-java-client-javadoc-3.6.1/com/rabbitmq/client/AMQP.BasicProperties.html#getTimestamp(). |

Table 3 - Message attributes according to AMQP

### Protobuf convention

Messages that contain the AMQP content-type attribute based on message types market/request, market/response and market/broadcast, include a binary-formatted protobuf (proto3) data section.

The binary format protobuf (proto3) definition uses the following conventions:

- MSG (MESSAGE): Used only in custom messages (e.g. AddOrderReq)

- FIELD: It describes custom value fields within the message (e.g. price), as well as value structures (e.g. order structure in the AddOrderReq message).

### Quantity values within messages

Quantity values in all messages are expressed as integer (int32). A custom value is defined by a field group in the message ProductInfoRprt – decimal_shift_quantity, min_quantity and quantity_unit (see chapter 2.8.4.13 Product Information Report (ProductInfoRprt)). The decimal_shift_quantity item defines the position of decimal point within the input integer (e.g. a quantity value of 5200 with field value decimal_shift_quantity = 3 equals 5.200 value).

The item min_quantity ensures the smallest possible step after entering the quantity (e.g. min_quantity = 100 and decimal_shift_quantity = 3 mean that it is possible to increase the quantity in steps of 0.1).

The item quantity_unit defines quantity unit.

### Price values within messages

Price values in all messages are expressed as integer (int64). A custom value is defined by a field group in the message ProductInfoRprt – decimal_shift_price, tick_size and currency (see chapter 2.8.4.13 Product Information Report (ProductInfoRprt)).

The decimal_shift_price item defines the position of decimal point within the input integer (e.g. a price value of 3624 with decimal_shift_price = 2 equals 36.24 value).

The item tick_size defines the smallest possible step after entering the price (e.g. tick_size = 1 and decimal_shift_price = 2 mean that it is possible to increase the price in steps of 0.01).

The item currency defines trade currency.

### Format of date items within messages

Date items within messages use the Timestamp data type (native google.protobuf.Timestamp), which uses the unix UTC timestamp as the base value (for more information see https://www.unixtimestamp.com/).

### Heartbeat message

The Heartbeat message contains text with the attributes “server-timestamp” and “interal-length”. Both attributes use milliseconds. The first one represents the difference between an actual time and the date 1.1.1970 0:00:00 UTC.

Message example: server-timestamp=1468251175238;interval-length=30000

### Standard message header

Every message contains the standard message header with the following items:

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| standard_header | FIELD | m |  | Structure |  |
| market_id | FIELD | m |  | Enum | Market Identification Code (MIC) of the market to which the request is sent or from which the request originates. The following values are allowed: “MARKET_ID_TYPE_XBID”: XBID Intraday market "MARKET_ID_TYPE_IM": OTE secondary Intraday market (fallback to XBID). |
| client_correlation_id | FIELD | o |  | String | The client data field in this section can be used by the client to store information or meta-data about a request. The content in this field is not used by CS OTE system. Content is sent back to client in response. |
| client_data_string | FIELD | o |  | String |  |

Table 4 - Message header

### Individual message parameter descriptions

The following chapters define the following message parameters:

- Type – message type

  - Inquiry Request – data request

  - Management request – performance request

  - Broadcast – mass message

- Role – role based accessibility

- Routing key –message routing to MQ server

- Message limit – max. number of name-specific messages that can be processed by the server for each user within the defined time limit, without being rejected by the server. The format is defined as a/b, where the first “a” represents max. number of messages allowed within a 1-minute time limit and “b” represents max. number of messages within a 1-hour time limit. If no time limit is set, the number of messages is unlimited. Limit is calculated separately for each market_id.

## Communication scenarios

### User log-in, log-out

The base communication scenario is used for user log-in, log-out access to the system as well as actual system information requests. The user must intiate data communication with MQ server by sending a log-in request LoginReq within a 30-second time limit, otherwise the connection will be terminated. The response is UserRprt returned as a result of a successful validation, otherwise ErrResp is sent to the client in case of an unsuccessful validation.

The user must send a LogoutReq message during client application termination. If the user does not send a log-out request, they will be logged out according to the rules that define a lost connection.

User

(request initiator)

OTE

LoginReq

ErrResp

UserRprt

LogoutReq

LogoutRprt

Request / response

From the client’s perspective, this message is synchronous. Client waits for the response.

Message sent as a distributed message.

Figure 3 - User login/logout sequence diagram

### Base scenario of bid submission – error processing

Bid creation or modification requests are formally validated and according to the control results, the user is either notified via a formal message receipt confirmation (AckResp) or formal error message (ErrResp). In case of a successful receipt (after sending AckResp to the user), the request validation follows on the OTE side and the XBID central side. In case of an unsuccessful validation, an ErrResp error notification is generated for the user, which is then sent as a distributed message.

User

(request initiator)

(request initiator)

OTE

OTE management request

Request / response

From the client’s perspective, this message is synchronous. Client waits for the response.

ErrResp (formal checks NOK)

AckResp

Message sent as a distributed message.

ErrResp (validation NOK)

Figure 4 - General request sequence diagram

### Bid manipulation

The user submits an bid using the AddOrderReq (or modifies the bid using the ModifyOrderReq) and the application server responds with an AckResp message, either confirming that the request was successfully received or informing the user about the error message definition via the ErrResp message. The server sends the submission/modification result message through a private OrderExecutionRprt message as well as via a MessageRprt private message.

Next, in case of a successful bid submission, the PublicOrderBooksDeltaRprt message, including the public order book modifications, is sent to all OTE users. This also applies in the case of a bid modification (or a set of bids), but only if such modification changes its timestamp.

In case of a trade establishment, the TradeCaptureRprt private message is sent to the bid owner, and the public MessageRprt message is sent to all OTE users. In case of a trade establishment between OTE users (intrastate Czech trade), private messages OrderExecutionRprt and TradeCaptureRprt are also sent to the counter-bid owner. In case of interstate trade the XBID central side ensures notifying the counter-bid owner outside of the Czech area.

There is also an option to submit a bid request via the OrderReq, where the response is sent in the form of a distributed private message OrderExecutionRprt from OTE back to the request initiator.

OTE

Users

(the whole OTE market)

AddOrderReq

AckResp

OrderExecutionRprt

Request / response

Message sent as a distributed message.

From the client’s perspective, this message is synchronous. Client waits for the response.

User

(request initiator)

MessageRprt

TradeCaptureRprt

MessageRprt

PublicTradeConfirmationRprt

PublicOrderBookDeltaRprt

If a trade is established

OrderReq

OrderExecutionRprt

Figure 5 - Order submission and trade establishment sequence diagram

OTE

Users

(the whole OTE market)

Market)

OrderModifyReq

AckResp

OrderExecutionRprt

Request / response

Message sent as a distributed message.

From the client’s perspective, this message is synchronous. Client waits for the response.

User

(request initiator)

MessageRprt

TradeCaptureRprt

MessageRprt

PublicTradeConfirmationRprt

If a trade is established

Only if execution priority is modified

PublicOrderBookDelta Rprt

Figure 6 - Order modification and trade establishment sequence diagram

If the submitted creation or modification request is invalid, the ErrResp message is sent as a response to the request initiator (see chapt. 2.7.2 Base scenario of bid submission – error ).

During a bulk bid modification (such as activation, deactivation and annulment) based on the ModifyAllOrdersReq user request, a message sequence similar to the one in case of bid submission or modification is applied, as described in the introduction section of this chapter, with the information that these messages contain information about the set of modified n bids. If there is a m amount of trades established (where the number of trades m ≤ number of bids n), then the relevant messages concerning trade establishment contain information about a set of m trades. If a set of m trades is created (where the number of trades m ≤ number of bids n), then the system will provide a set of m relevant trade establishment messages.

OTE

Users

(the whole OTE market)

ModifyAllOredrsReq

AckResp

OrderExecutionRprt (n orders)

Request / response

Message sent as a distributed message.

From the client’s perspective, this message is synchronous. Client waits for the response.

User

(request initiator)

MessageRprt (n messages)

TradeCaptureRprt (m trades)

MessageRprt (m messages)

PublicTradeConfirmationRprt (m trades)

PublicOrderBookDeltaRprt

If a trade is established or if trades are established

Only if market depth is modified

Figure 7 - Bulk order modification (or deactivation) and the subsequent order request sequence diagram

### Trade callback

Note: The trade callback communication scenario is not available.

The user can submit a trade callback request via the TradeCallbackReq message. If the request is formally valid, the AckResp is sent to the user, otherwise the ErrResp containing the error specification is sent. After the internal request processing, this request is forwarded to the XBID central side.

XBID then processes the request and marks the trade as cancelled by changing its status, thereby notifying OTE, who then updates the trade status according to XBID and informs the request intiator via the TradeCaptureReprt message as well as the audit log MessageRprt message. All OTE users are also notified via PublicTradeConfirmationRprt.

After trade callback request process is completed on the central side, XBID informs OTE about the trade callback result, which is then reflected by OTE through a trade update change according to XBID. The request initiator is then informed again via the TradeCaptureRprt message as well as the audit log MessageRprt, which also informs about the trade callback result. All OTE users are informed about the result too via the PublicTradeConfirmationRprt message.

TradeRecallReq

OTE

AckResp

TradeCaptureRprt

Request / response

Message sent as a distributed message.

From the client’s perspective, this message is synchronous. Client waits for the response.

User

(request initiator)

MessageRprt

TradeCaptureRprt

MessageRprt

PublicTradeConfirmationRprt

PublicTradeConfirmationRprt

Users

(trade sides)

Users

(the whole OTE market)

ErrResp

Figure 8 - Trade recall sequence diagram

### Trade cancellation

The process of initiatiating and cancelling a trade is performed within the central XBID solution, all NEMO are only informed about the trade cancellation. If at least one side of the cancelled trade is a OTE market participant, then OTE sends the TradeCaptureRprt and the MessageRprt distributed message to the cancelled trade market participant (if the cancelled trade is local, then both market participants will receive the message). All users are notified by OTE about the trade cancellation by sending the PublicTradeConfirmationRprt public message. This does not include cancelled trades, where both sides are not OTE users.

Request / response

Message sent as a distributed message.

From the client’s perspective, this message is synchronous. Client waits for the response.

Trade cancellation event from XBID

OTE

TradeCaptureRprt

MessgeRprt

PublicTradeConfirmationRprt

Users

(trade sides)

Users

(the whole OTE market)

If the cancelled trade relates to the CZ area

Figure 9 - Trade cancellation sequence diagram

### Public bid data request

The user sends a request for the list of active public market bid through PublicOrderBooksReq and the server responds with the PublicOrderBooksResp containg a copy of these bids. This is how the client receives the entire set of active bids withing the system. If a completely new bid is  created or already existing bid is  modified, the PublicOrderBooksDeltaRprt mass message will be sent.

User

(request initiator)

OTE

PublicOrderBooksReq

PublicOrderBooksResp

PublicOrderBooksDeltaRprt

PublicOrderBooksDeltaRprt

PublicOrderBooksDeltaRprt

Request / response

Message sent as a distributed message.

All order modifications are distributed to all users.

Figure 10 - Order request sequence diagram

### Public trade data request

The user sends a request for established market trades through the PublicTradeConfirmationReq and the server responds with the PublicTradeConfirmationRprt containing a copy of these trades. The case of a trade establishment is illustrated in the following messages.

User

(request initiator)

OTE

PublicTradeConfirmationReq

PublicTradeConfirmationRprt

PublicTradeConfirmationRprt

PublicTradeConfirmationRprt

PublicTradeConfirmationRprt

Request / response

Message sent as a distributed message.

Figure 11 - Trade request sequence diagram

### Information message request

After the successful log-in of the user, the user then sends the MessageReq request to the server inquiring about a message list. The user may specify within this request whether they would like to receive only private messages, only public messages or all messages. They will be delivered within the required time limit via the MessageRprt message and all the following messages are then distributed automatically.

User

(request initiator)

OTE

MessageReq

MessageRprt

MessageRprt

Request / response

Message sent as a distributed message.

MessageRprt

Figure 12 - Market messages request sequence diagram

### Product and market contract request

The user may request the list of valid products via the ProductInfoReq request and the response is sent in the message ProductInfoRprt. In case of a product modification, the ProductInfoRprt distributed public message is sent to all OTE users.

Similar situation occurs in case of the Contract information. The user may request the list of valid contract via the ContractInfoReq and the response is delivered through the message ContractInfoRprt. In case of a contract modification, all OTE users will receive the ContractInfoRprt public distributed message.

User

(request initiator)

OTE

ProductInfoReq

ProductInfoRprt

ContractInfoRprt

Request / response

Message sent as a distributed message.

ProductInfoRprt

ContractInfoReq

ContractInfoRprt

Users

(the whole OTE market)

Figure 13 - Product and contract request sequence diagram

### Market status request

The user may request the current market status information through the MarketStateReq request and the response will be sent via the MarketStateRprt message. In case of a market status modification, the MarketStateRprt public distributed message will be sent to all OTE users. These messages allow monitoring of the current market status, to ensure that it is not in the “Deactivated” state where trading is stopped.

OTE

MarketStateReq

Users

(the whole OTE market)

MarketStateRprt

Request / response

Message sent as a distributed message.

MarketStateRprt

User

(request initiator)

Figure 14 - Market state request sequence diagram

### Capacity data request

The user may request the current H2H matrix status via the HubToHubReq request and the response is sent as the HubToHubResp distributed message, whose structure is similar to the HubToHubNtfRprt distributed message.

All the delta changes of the capacity data values represented by the H2H matrix are automatically distributed to all users via the HubToHubNtfRprt distributed message.

OTE

HubToHubReq

Users

(the whole OTE market)

HubToHubNtfRprt

Request / response

Message sent as a distributed message.

HubToHubResp

User

(request initiator)

Figure 15 - H2H matrix request sequence diagram

### Market area request

Users may request market area information via the MarketAreaInfoReq, the MarketAreaInfoRprt is sent as a response to the initiator of this request.

OTE users are informed via the MarketAreaInfoRprt distributed message, in case any attribute within the market area is modified.

OTE

MarketAreaInfoReq

MarketAreaInfoRprt

Request / response

Message sent as a distributed message.

MarketAreaInfoRprt

User

(request initiator)

Users

(the whole OTE market)

Figure 16 - Market area request sequence diagram

### Delivery area request

Users may request delivery area data via the DeliveryAreaInfoReq message too, the DeliveryAreaInfoRprt is sent to the initiator as a response.

OTE users are infomed via the DeliveryAreaInfoRprt distributed message in case any attribute within the delivery area is modified.

OTE

DeliveryAreaInfoReq

DeliveryAreaInfoRprt

Request / response

Message sent as a distributed message.

DeliveryAreaInfoRprt

User

(request initiator)

Users

(the whole OTE market)

Figure 17 - Delivery area request sequence diagram

### Market state distributed message

An informative message about the market state, distributed from XBID to OTE during market state changes (ACTI->HIBE and back) and sent from OTE to market participants.

OTE

MarketStateRprt

Message sent as a distributed message.

Users

(celý trh)

Figure 18 - Market state distributed message sequence diagram

## Communication messages

The content of all messages exchanged between the user and IM application within the above mentioned communication scenarios is in binary format protobuf. A detailed description of all messages is provided in the following chapters.

Modification summary compared to the default XML interface:

- Some BINARY API specifications are identical to the verified protobuf (proto3) procedure recommendations, for example:

  - names of enum values, for example enum value of the item validity_restriction = “VALIDITY_RESTRICTION_TYPE_GFS”, use the name of the common enum data type as a prefix – in this example it is “ValidityRestrictionType”

  - each  enum type of protobuf definition  includes a dedicated enum value “_UNSPECIFIED”, which the protobuf framework interprets as an implicit value if the current item is empty

  - Items with timestamp and duration values use the corresponding pre-built protobuf data types.

- The naming of certain messages has been modified to increase clarity and readability (for example from the PblcTradeConfRprt message to the PublicTradeConfirmationRprt). The following rules apply to the message name postfix:

  - Req postfix – data request message

  - Resp postfix – response message to a request

  - Rprt postfix – distributed message or request response message

- Each message item is, in some cases, renamed to improve clarity and reability, using the current protobuf (proto3) naming recommendations:

  - Items follow the lower_snake_case naming convention , where all letters are lowercase and words are separated by an underscore

  - The enum values of the enum-type items follow the CAPITALS_WITH_UNDERSCORES naming convention

  - In case where items contain a value field (with cardinality > 1), the name uses the English plural form with the letter “s” at the end (for example the structure bids within the AddOrderReq message)

- Message cleanup – certain obsolete items have been removed, such as:

  - Items that include field encapsulation into different structures, for example: OrdrlList, MktAreaList, MsgList etc.

  - Removal of a clientData structure from the header of all standard_header messages, with the exception of retaining one item from this structure – client_correlation_id, which is now placed at the same level as the market_id item

The order of items in individual messages is determined by the definition protobuf(.proto) file, see [4]. The order of items specified in the AK manual is not binding.

Note: modifications compared to the original XML format in the naming of messages, items, data types and enum types are not represented in this document. Nevertheless these modifications compared to the original XML format are evident from the document [3], which maps the OTECOM message items from the original XML format to the new protobuf (proto3) format with changes to the message, item, data type and enum type names. Modifications are highlighted in red.

### General requests and responses

#### Login Request (LoginReq)

| LoginReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | <All> |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 3/20 |

Login request. The system responds via the UserRprt.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| LoginReq | MSG |  |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| user | FIELD | m |  | String | Login ID of the user that want to login to the CS OTE system. |
| force | FIELD | m |  | Boolean | Flag that indicates if this user wants to force a login even if a user with the same credentials is already logged in into the CS OTE system. |
| disconnect_action | FIELD | m |  | Enum | Action that will be executed in case of an unexpected connection loss between user and CS OTE system, irrespective of where the connection loss will be (user – AMQP – CS OTE system). The following values are allowed: "DISCONNECT_ACTION_TYPE_NO": No action is executed. "DISCONNECT_ACTION_TYPE_DEACT_USER_ORDERS": All orders of this user will be deactivated. |

Table 5 - Login request message structure

#### User Report (UserRprt)

| UserRprt |  |
| --- | --- |
| Type: | Management Response, Broadcast |
| Response to: | LoginReq (sent to the user-generated private response queue or a broadcast to market.broadcastQueue.<login-id>) |
| Broadcasted: | Yes |
| Broadcast Routing Keys: | USR_<login-id> |
| Roles: | <All> |

This message contains the basic user attributes. The UserRprt message is returned as a response to the LoginReq and is also distributed when a configuration modification of a user’s product assignment occurs.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| UserRprt |  | MSG |  |  | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| session_id |  | FIELD | m |  | Integer(64) | The current session id of the user given after login to the system. |
| connection_loss_message |  | FIELD | o |  | String | In case of a connection loss for the previous user session, this field is filled with a connection loss message, indicating the connection loss event with date and time and the logout action executed by the CS OTE system. |
|  | name | FIELD | m |  | String | Name of the user. |
|  | partic_name | FIELD | m |  | String | Participant name. |
|  | partic_id | FIELD | m |  | Integer(64) | The participant id the user belongs to. |
|  | state | FIELD | m |  | Enum | Current state of the User. The following values are allowed: "REFERENCE_DATA_STATE_TYPE_ACTI": User is active. It is possible to trade using this User. "REFERENCE_DATA_STATE_TYPE_DELE": User is deleted. Trading using this User is not possible. "REFERENCE_DATA_STATE_TYPE_SUSP": User is suspended. Trading using this User is not possible. |
|  | user_roles | FIELD | m | 1..n | String | Contains the user roles assigned to the user |
|  | user_id | FIELD | m |  | Integer(64) | The unique identifier of a user. |
| assigned_markets |  | FIELD | m | 1..n | Structure |  |
|  | market_id | FIELD | m |  | Enum | Market Identification Code. The following values are allowed: “MARKET_ID_TYPE_XBID”: XBID Intraday market "MARKET_ID_TYPE_IM": OTE secondary Intraday market (fallback to XBID). |
|  | default_delivery_area_id | FIELD | m |  | String | Delivery Area ID. |

Table 6 - User report message structure

#### Logout Request (LogoutReq)

| LogoutReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | <All> |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 3/20 |

The CS OTE system user logout request.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| LogoutReq | MSG |  |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| session_id | FIELD | m |  | Integer(64) | Session id of the client session passed to the client on login. |

Table 7 - Logout request message structure

#### Logout Report (LogoutRprt)

| LogoutRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | LogoutReq (sent to the user-generated private response queue or a broadcast to market. broadcastQueue.<login-id>) |
| Broadcast: | Yes |
| Broadcast Routing Keys: | USR_<login-id> |
| Roles: | <All> |

This message indicates a user logout from the CS OTE system. It is sent either as a response to the logout request LogoutReq or as a mass message triggered by a concurrent forced login of the same user (force=true).

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| LogoutRprt | MSG |  |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| session_id | FIELD | m |  | Integer(64) | Session id of the client session passed to the client on login. |
| user_id | FIELD | m |  | Integer(64) | User ID identification. |
| text | FIELD | o |  | String | Text field containing information about the reason of the logout. |

Table 8 - Logout report message structure

#### Acknowledgement Response (AckResp)

| AckResp |  |
| --- | --- |
| Type: | Management Response |
| Response to: | AddOrderReq; ModifyOrderReq; ModifyAllOrdersReq: (sent to the user-generated private response queue) |
| Broadcast: | No |
| Routing Keys: | --- |
| Roles: | <All> |

A confirmation message indicating that the request has been accepted for processing.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| AckResp | MSG |  |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |

Table 9 -  Acknowledgement response message structure

#### Error Response (ErrResp)

| ErrResp |  |
| --- | --- |
| Type: | Inquiry Response; Management Response; Broadcast |
| Response to: | <All> (sent to the user-generated private response queue or a broadcast to market. broadcastQueue.<login-id>) |
| Broadcast: | Yes |
| Broadcast Routing Keys: | USR_<login-id> |
| Roles: | <All> |

An error message distributed in case of unsuccessful processing of a request or inquiry.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| ErrResp |  | MSG |  |  | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| errors |  | FIELD | m | 1..n | Structure |  |
|  | error_code | FIELD | m |  | Integer | Predefined error codes. Some error messages do not have a specific error code. In this case the value is 0. |
|  | error_en | FIELD | m |  | String | The error message for this error – English version. |
|  | error_cz | FIELD | m |  | String | The error message for this error – Czech version. |
|  | client_order_id | FIELD | o |  | String | Client order ID. |

Table 10 - Error response message structure

### Bid submission and management

#### Add Order Request (AddOrderReq)

| AddOrderReq |  |
| --- | --- |
| Type: | Management Request |
| Roles: | EmtasImIns |
| Routing Keys: | market.request.management |

The submission of one or more bids. The maximum number of bids within a single message is 25. The message must be encapsulated and signed using the SignedMessage message, see chapt. 3 Using the electronic signature.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| AddOrderReq |  | MSG | m | 1 | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| list_execution_instruction |  | FIELD | o |  | Enum | Defines the execution instruction for the whole list of orders: “LIST_EXECUTION_INSTRUCTION_TYPE_LNKD”: Linked orders - the provided orders are linked together and should be executed all at once. This option can only be used, if all orders have execution restriction ORDER_EXECUTION_RESTRICTION_TYPE_FOK (Fill or Kill). Orders can be submitted for Contracts of different Products. In case one of the orders cannot be executed, the whole list is not executed. The Linked Orders feature is configurable and might be turned off. “LIST_EXECUTION_INSTRUCTION_TYPE_NONE”: All orders are treated independently. This is the Default Value. “LIST_EXECUTION_INSTRUCTION_TYPE_VALID”: All orders must be valid, meaning they must past the order validation of the XBID system (e.g. the price of the order must be in the price range of the product). If one order does not pass the validation, the full list of submitted orders is rejected. |
| orders |  | FIELD | m | 1.. 100 | Structure |  |
|  | state | FIELD | o |  | Enum | “ORDER_ENTRY_STATE_TYPE_ACTI”: The order is entered and immediately exposed to the market for execution. This is the default value. “ORDER_ ENTRY_STATE_TYPE_HIBE”: The order is entered into the CS OTE system but not exposed to the market. |
|  | validity_restriction | FIELD | o |  | Enum | Validity restriction of the order. If this field is omitted, the order will be treated as a “Good for Session” order. Valid values: “VALIDITY_RESTRICTION_TYPE_GFS” (Good for trading session): The order rests in the order book until it is either executed, removed by the user or the current trading session (trading phase) of the underlying contract ends. “VALIDITY_RESTRICTION_TYPE_GTD”. The order rests in the order book until the date specified in the validity_date field. “VALIDITY_RESTRICTION_TYPE_NON” (No validity restriction): Mandatory for orders with the execution restriction “ORDER_EXECUTION_RESTRICTION_TYPE_NON FOK” or “ORDER_EXECUTION_RESTRICTION_TYPE_NON IOC”. |
|  | validity_date | FIELD | o |  | Timestamp | This field is mandatory in case of validityRes equals “VALIDITY_RESTRICTION_TYPE_GTD”. It is used to define the date until which the order is valid. The remaining part of the order will be removed from the order book after this point in time. |
|  | text | FIELD | o |  | String | Comment entered by the user. Maximum possible length is 250 characters. |
|  | type | FIELD | m |  | Enum | Order type. Valid values: “ORDER_TYPE_O”: Regular limit order (for all predefined contracts). “ORDER_TYPE_I”: Iceberg order. “ORDER_TYPE_B”: User defined block order. |
|  | client_order_id | FIELD | o |  | String | Client Order Id with a maximum length of 40 characters. |
|  | delivery_area_id | FIELD | m |  | String | Defines the delivery area of the order.The delivery_area_id is respecting codes provided by DeliveryAreaInfoRprt. |
|  | order_execution_restriction | FIELD | o |  | Enum | Execution restriction of the order. Valid values: “ORDER_EXECUTION_RESTRICTION_TYPE_NON”: No restriction. This is the default. “ORDER_EXECUTION_RESTRICTION_TYPE_FOK” (Fill or Kill): The order is immediately fully executed or deleted. “ORDER_EXECUTION_RESTRICTION_TYPE_IOC” (Immediate and Cancel): The order is executed immediately to its maximum extend. In case of a partial execution, the remaining volume is removed from the order book. “ORDER_EXECUTION_RESTRICTION_TYPE_AON” (All or None): The order must be filled completely or not at all. The order stays in the order book until it is executed or removed by the system or user. This execution restriction can be used only in combination with User Defined Block Orders (for which only ORDER_EXECUTION_RESTRICTION_TYPE_AON execution restriction is allowed). |
|  | quantity | FIELD | m |  | Integer | Contains the total quantity of the order. In case of an Iceberg order this field corresponds to the hidden quantity + display quantity. |
|  | display_quantity | FIELD | o |  | Integer | Used to define display quantity of an Iceberg Order. This field is required only in the case of type=’ORDER_TYPE_I’. |
|  | price | FIELD | m |  | Integer(64) | Limit price of the order in currency defined by contracts. Value is multiplied by 100, e.g. 1 Euro = 100. |
|  | side | FIELD | m |  | Enum | Defines on which side of the market the order is entered (“DIRECTION_TYPE_BUY”, “DIRECTION_TYPE_SELL”). |
|  | product_name | FIELD | o |  | String | Product identifier. Required in case of the (long name) is omitted. |
|  | contract | FIELD | o |  | String | Contract code identifier (long name). Applicable for orders for pre-defined contracts only. |
|  | delivery_start | FIELD | o |  | Timestamp | Start of delivery of the underlying contract. Applicable for User Defined Block Orders only. The field is ignored if contract is filled in. |
|  | delivery_end | FIELD | o |  | Timestamp | End of delivery of the underlying contract. Applicable for User Defined Block Orders only. The field is ignored if contract is filled in. |
|  | peak_price_delta | FIELD | o |  | Integer(64) | Peak price delta for Iceberg orders. - The peak_price_deltaof buy orders must be smaller or equal than zero. - The peak_price_deltaof sell orders must be greater or equal than zero. If it is omitted the system will assume a value of “0,00”. |

Table 11 - Add order request message structure

#### Order Modify Request (ModifyOrderReq)

| ModifyOrderReq |  |
| --- | --- |
| Type: | Management Request |
| Roles: | EmtasImIns |
| Routing Keys: | market.request.management |

The modification message for one or more bids. The maximum number of bids within a single message is 25. In the case of an bid activation or deactivation request in the XBID market, modification of other attributes is disabled. The message must be encapsulated and signed using the SignedMessage message, see chapt. 3 Using the electronic signature.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| ModifyOrderReq |  | MSG | m | 1 | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| modify_order_type |  | FIELD | m |  | Enum | Offers the possibility to activate, deactivate, modify or delete all orders contained in the basket. “MODIFY_ORDER_TYPE_ACTI”: Activate all orders contained in this basket. Already active orders are ignored. “MODIFY_ORDER_TYPE_HIBE”: Deactivates (hibernates) all orders contained in the basket. Hibernated orders are removed from the order book but are still available for modification or activation in the own orders list. “MODIFY_ORDER_TYPE_MODI”: Modifies all orders in the basket. “MODIFY_ORDER_TYPE_DELE”: Deletes all orders in the basket. |
| orders |  | FIELD | m | 1..25 | Structure | List of single order definitions. |
|  | revision_no | FIELD | m |  | Integer(64) | The latest revision number of the order must be provided by the user. In case the CS OTE has another revision number of currently valid order, it will reject the request with an ErrResp. |
|  | validity_restriction | FIELD | o |  | Enum | Validity restriction of the order. If this field is omitted, the order will be treated as a “Good for Session” order. Valid values: “VALIDITY_RESTRICTION_TYPE_ GFS” (Good for trading session): The order rests in the order book until it is either executed, removed by the user or the current trading session (trading phase) of the underlying contract ends. “VALIDITY_RESTRICTION_TYPE_ GTD” (Good till date): The order rests in the order book until the date specified in the validityDate field. “VALIDITY_RESTRICTION_TYPE_ NON” (No validity restriction): Mandatory for orders with the execution restriction “ORDER_EXECUTION_RESTRICTION_TYPE_FOK” or “ORDER_EXECUTION_RESTRICTION_TYPE_IOC”. |
|  | validity_date | FIELD | o |  | Timestamp | This field is mandatory in case of validity_restriction equals “VALIDITY_RESTRICTION_TYPE_GTD”. It is used to define the date until which the order is valid. The remaining part of the order will be removed from the order book after this point in time. |
|  | type | FIELD | m |  | Enum | Order type. Valid values: “ORDER_TYPE_O”: Regular limit order (for all predefined contracts). “ORDER_TYPE_I”: Iceberg order. “ORDER_TYPE_B”: User defined block order. Order type cannot be changed by modification to or from order type “ORDER_TYPE_B”. |
|  | text | FIELD | o |  | String | Comment entered by the user. Maximum possible length is 250 characters. |
|  | order_execution_restriction | FIELD | o |  | Enum | Execution restriction of the order. Valid values: “ORDER_EXECUTION_RESTRICTION_TYPE_NON”: No restriction. This is the default. “ORDER_EXECUTION_RESTRICTION_TYPE_FOK” (Fill or Kill): The order is immediately fully executed or deleted. “ORDER_EXECUTION_RESTRICTION_TYPE_IOC” (Immediate and Cancel): The order is executed immediately to its maximum extend. In case of a partial execution, the remaining volume is removed from the order book. “ORDER_EXECUTION_RESTRICTION_TYPE_AON” (All or None): The order must be filled completely or not at all. The order stays in the order book until it is executed or removed by the system or user. This execution restriction can be used only in combination with User Defined Block Orders (for which only ORDER_EXECUTION_RESTRICTION_TYPE_AON execution restriction is allowed). |
|  | quantity | FIELD | m |  | Integer | Contains the total quantity of the order. In case of an Iceberg order this field corresponds to the hidden quantity + display quantity. |
|  | display_quantity | FIELD | o |  | Integer | Used to define display quantity of an Iceberg Order. |
|  | price | FIELD | m |  | Integer(64) | Limit price of the order in currency defined by contract. Value is multiplied by 100, e.g. 1 Euro = 100. |
|  | client_order_id | FIELD | o |  | String | client_order_id with a maximum length of 40 characters. |
|  | order_id | FIELD | m |  | Integer(64) | Order Id as returned by the CS OTE system. This value is used to identify the order to be modified. |
|  | peak_price_delta | FIELD | o |  | Integer(64) | Peak price delta for Iceberg orders. - The peak_price_delta of buy orders must be smaller or equal than zero. - The p peak_price_delta pd of sell orders must be greater or equal than zero. If it is omitted the system will assume a value of “0,00”. |

Table 12 - Order modify request structure

#### Order Request (OrderReq)

| OrderReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 10/30 |

A custom bid status request.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| OrderReq | MSG | m | 1 | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| contracts | FIELD | o | 0.. 1000 | String | List of contract codes (long name). If no contract code is given, the own orders for all contracts assigned to the requesting user are returned. |

Table 13 - Order request message structure

#### Order Execution Report (OrderExecutionRprt)

| OrderExecutionRprt |  |
| --- | --- |
| Type: | Management Response; Broadcast |
| Response to: | AddOrderReq; ModifyOrderReq; OrderReq; ModifyAllOrdersReq; (sent to the user-generated private response queue or a broadcast to market.broadcastQueue.<login-id>) |
| Broadcast: | Yes |
| Broadcast Routing Keys: | <product_name>.PRTC_<partic_id> |
| Roles: | EmtasImTsAcc |

A message indicating a successful bid modification. This message is sent to market participants in the following cases:

- A successful bid submission,

- A successful bid modification,

- A partially or fully traded bid,

As a response to an bid request (in this case, it is sent to the private response queue, in all other cases it is sent to the mass message queue).

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| OrderExecutionRprt |  | MSG | m | 1 | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| list_execution_instruction |  | FIELD | o |  | Enum | Defines the execution instruction for the whole list of orders: “LIST_EXECUTION_INSTRUCTION_TYPE_LNKD”: Linked orders - the provided orders are linked together and should be executed all at once. This option can only be used, if all orders have execution restriction. “LIST_EXECUTION_INSTRUCTION_TYPE_NONE”: All orders are treated independently. This is the Default Value. “LIST_EXECUTION_INSTRUCTION_TYPE_VALID”: All orders must be valid, meaning they must past the order validation of the XBID system (e.g. the price of the order must be in the price range of the product). If one order does not pass the validation, the full list of submitted orders is rejected. |
| orders |  | FIELD | o | 0..n | Structure |  |
|  | action | FIELD | m |  | Enum | Code of the last action provided on the order. Valid values are: “ORDER_ACTION_TYPE_UADD”: Order added by user. “ORDER_ACTION_TYPE_UHIB”: Order hibernated by user. “ORDER_ACTION_TYPE_UMOD”: Order modified by user. “ORDER_ACTION_TYPE_UDEL”: Order deleted by user. “ORDER_ACTION_TYPE_SHIB”: Order hibernated by the system. “ORDER_ACTION_TYPE_SMOD”: Order modified by the system. “ORDER_ACTION_TYPE_SDEL”: Order deleted by the system. “ORDER_ACTION_TYPE_FEXE”: Order is fully executed. If an order comes into the system and gets executed immediately by matching an already existing order only one OrderExecutionRprt for this order is sent with action ORDER_ACTION_TYPE_FEXE or ORDER_ACTION_TYPE_PEXE. If an order comes into the system and gets executed by a later entered order two messages are sent. One for the order entry with ORDER_ACTION_TYPE_UADD and later one for the execution with either ORDER_ACTION_TYPE_FEXE or ORDER_ACTION_TYPE_PEXE. “ORDER_ACTION_TYPE_PEXE”: Partial execution of order. “ ORDER_ACTION_TYPE_IADD”: A new slice of an Iceberg order was added to the service. |
|  | validity_restriction | FIELD | o |  | Enum | Validity restriction of the order. If this field is omitted, the order will be treated as a “Good for Session” order. Valid values: “VALIDITY_RESTRICTION_TYPE_GFS” (Good for trading session): The order rests in the order book until it is either executed, removed by the user or the current trading session (trading phase) of the underlying contract ends. “VALIDITY_RESTRICTION_TYPE_GTD” (Good till date): The order rests in the order book until the date specified in the vldtyDate field. “VALIDITY_RESTRICTION_TYPE_NON” (No validity restriction): Mandatory for orders with the execution restriction “ORDER_EXECUTION_RESTRICTION_TYPE_FOK” or “ORDER_EXECUTION_RESTRICTION_TYPE_IOC”. |
|  | validity_date | FIELD | o |  | Timestamp | This field is mandatory in case of validity_restriction equals “VALIDITY_RESTRICTION_TYPE_GTD”. It is used to define the date until which the order is valid. The remaining part of the order will be removed from the order book after this point in time. |
|  | timestamp | FIELD | m |  | Timestamp | Timestamp of the order entry as determined by the CS OTE system. This timestamp determines the execution priority in case of identical limit prices. |
|  | revision_no | FIELD | m |  | Integer(64) | This value is increased in case of a partial execution, hibernation, modification without execution priority change. |
|  | user_id | FIELD | m |  | Integer(64) | User id of the user who entered the order. |
|  | state | FIELD | m |  | Enum | The current state of the order in the system. Valid values: “ORDER_STATE_TYPE_HIBE”: The order is entered into the XBID SOB system but not exposed to the market. “ORDER_STATE_TYPE_ACTI”: The order is entered and immediately exposed to the market for execution “ORDER_STATE_TYPE_IACT”: The order is inactive due time validity or fully executed. “ORDER_STATE_TYPE_DELE”: The order is deleted |
|  | type | FIELD | m |  | Enum | Order type. Valid values: “ORDER_TYPE_O”: Regular limit order (for all predefined contracts). “ORDER_TYPE_I”: Iceberg order. “ORDER_TYPE_B”: User defined block order. |
|  | client_order_id | FIELD | o |  | String | LTS Order Id with a maximum length of 40 characters. This value is not modified by the CS OTE system and may be used by LTS to identify orders. |
|  | delivery_area_id | FIELD | m |  | String | Defines the delivery area of the order.The delivery_area_id is respecting codes provided by DeliveryAreaInfoRprt. |
|  | text | FIELD | o |  | String | Comment entered by the user. Maximum possible length is 250 characters. |
|  | order_execution_restriction | FIELD | o |  | Enum | Execution restriction of the order. Valid values: “ORDER_EXECUTION_RESTRICTION_TYPE_FOK” (Fill or Kill): The order is immediately fully executed or deleted. “ORDER_EXECUTION_RESTRICTION_TYPE_IOC” (Immediate and cancel): The order is executed immediately to its maximum extend. In case of a partial execution, the remaining volume is removed from the order book. “ORDER_EXECUTION_RESTRICTION_TYPE_NON”: No restriction. “ORDER_EXECUTION_RESTRICTION_TYPE_AON” (All or None): The order must be filled completely or not at all. The order stays in the order book until it is executed or removed by the system or user. ORDER_EXECUTION_RESTRICTION_TYPE_AON execution restriction can be used only in combination with User Defined Block Orders (for which only ORDER_EXECUTION_RESTRICTION_TYPE_AON execution restriction is allowed) and hence can’t be changed by modification. |
|  | initial_quantity | FIELD | m |  | Integer | The total quantity entered with this order. If the order is partially matched, the initial_quantity still contains the original quantity value. |
|  | quantity | FIELD | m |  | Integer | Contains the quantity exposed to the market. In case of an Iceberg Order this is the rest of the display quantity. |
|  | hidden_quantity | FIELD | o |  | Integer | Contains the hidden quantity of the Iceberg order. The total executable quantity may be calculated by adding the hidden_quantity to the quantity. |
|  | display_quantity | FIELD | o |  | Integer | Used to define display quantity of an Iceberg Order. |
|  | price | FIELD | m |  | Integer(64) | Limit price of the order in currency defined by contract. Value is multiplied by 100, e.g. 1 Euro = 100. |
|  | side | FIELD | m |  | Enum | Defines on which side of the market the order is entered. Valid values: “DIRECTION_TYPE_BUY”: Buy order. “DIRECTION_TYPE_SELL”: Sell order. |
|  | contract | FIELD | m |  | String | Contract code identifier (long name). |
|  | initial_order_id | FIELD | m |  | Integer(64) | In case of an order modification, this value contains the Id of the first order in the modification chain. |
|  | parent_order_id | FIELD | o |  | Integer(64) | In case of an order modification this field contains the Id of the modified order. |
|  | order_id | FIELD | m |  | Integer(64) | Order Id as returned by the CS OTE system. |
|  | last_update_user_id | FIELD | m |  | Integer(64) | Information about the user who last updated the order |
|  | peak_price_delta | FIELD | o |  | Integer(64) | Peak price delta for Iceberg orders. |

Table 14 - Order execution report message structure

#### Modify All Orders Request (ModifyAllOrdersReq)

| ModifyAllOrdersReq |  |
| --- | --- |
| Type: | Management Request |
| Roles: | EmtasImIns |
| Routing Keys: | market.request.management |

A message for a mass activation, deactivation and bid cancellation. The message must be encapsulated and signed using the SignedMessage, see chapt. 3 Using the electronic signature.

| Message/Field | Type | m/o | No. | Data Type | Short description |  |
| --- | --- | --- | --- | --- | --- | --- |
| ModifyAllOrdersReq | MSG |  |  | Structure |  |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |  |
| partic_id | FIELD | o |  | Integer(64) | Unique identifier of a partic. | One and only one of these fields must be supplied. |
| user_id | FIELD | o |  | Integer(64) | Unique identifier of a user. |  |
| order_modification_type | FIELD | m |  | Enum | Modification type for the orders: “MODIFY_ORDER_ALL_TYPE_ACTI”: Activate all orders. Already active orders are ignored. “MODIFY_ORDER_ALL_TYPE__HIBE”: Deactivates (hibernates) all orders. Hibernated orders are removed from the order book but are still available for modification or activation in the own orders list. “MODIFY_ORDER_ALL_TYPE_DELE”: Deletes all orders. |  |
| product_names | FIELD | o | 0.1000 | String | Only orders for products with the given product names will be modified. |  |
| delivery_area_ids | FIELD | o | 0..1000 | String | Orders for the given user_id and list of DA’s in delivery_area_id will be Deactivated or Deleted. This field can only be supplied when user_id is provided in the message. The delivery_area_id is respecting codes provided by DeliveryAreaInfoRprt. |  |
| contracts | FIELD | o | 0.. 1000 | String | List of contract codes (long name). If no contract code is given, the own orders for all contracts assigned to the specified participant or user are changed. |  |

Table 15 - Modify all orders request message structure

### IM Trade management

#### Trade Recall Request (TradeRecallReq)

| TradeRecallReq |  |
| --- | --- |
| Type: | Management Request |
| Roles: | EmtasImIns |
| Routing Keys: | market.request.management |

This message is used to submit a trade recall request. The request may only be submitted by the participant who owns an bid on at least one side of the trade, for both national and international (cross-border) trades. It contains only an identifier and a trade version.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| TradeRecallReq | MSG | m |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| trade_id | FIELD | m |  | Integer(64) | Trade Id of the trade to be recalled. |
| revision_no | FIELD | m |  | Integer(64) | The latest revision number of the trade must be provided by the MP. In case the OTE-COM system has another revision number, it will reject the request with an ErrResp. |

Table 16 - Trade recall request message structure

Note: The trade recall communication scenario is not available.

### Market information

#### Public Order Books Request (PublicOrderBooksReq)

| PublicOrderBooksReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | <All> |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 10/40 |

A public order book request for the specified contract.

| Message/Field | Type | m/o | No. |  |  | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PublicOrderBooksReq | MSG |  | 1 |  |  | Structure |  |
| standard_header | FIELD | m |  |  |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| contract_type | FIELD | (m) |  |  |  | Enum | Defines which kind of contracts should be retrieved: Possible values are: “CONTRACT_TYPE_ ALL” – All kind of contracts (pre-defined and user-defined) “CONTRACT_TYPE_ PDC” – Only pre-defined contracts “CONTRACT_TYPE_ UDC” – Only user-defined contracts This field is ignored when contracts field is specified. |
| product_names | FIELD | (m) |  | 0.. 1000 |  | String | List of product names. All order books for these products are returned. Delivery area may be specified to filter the result. Please note: If no product name is given, at least one contract (see below) must be provided. |
| contracts | FIELD | (m) |  | 0.. 1000 |  | String | List of contract codes (long name). Please note: If no contract is given, at least one product name (see above) must be provided. If both values are given the contract is taken. |
| delivery_area_ids | FIELD | O | 0.. 1000 |  |  | String | List of delivery areas for which the order book(s) should be retrieved. |

Table 17 - Public order books request message structure

#### Public Order Books Response (PublicOrderBooksResp)

| PublicOrderBooksResp |  |
| --- | --- |
| Type: | Inquiry Response |
| Response to: | PublicOrderBooksReq (sent to the user-generated private response queue) |
| Broadcast: | No |
| Broadcast Routing Keys: | --- |
| Roles: | <All> |

Public information about the current bids for the specified contract. The message is distributed as a response to the PublicOrderBooksReq request.

| Message/Field |  |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PublicOrderBooksResp |  |  | MSG | m | 1 | Structure |  |
| standard_header |  |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| order_books |  |  | FIELD | o | 0..n | Structure |  |
|  | revision_no |  | FIELD | m |  | Integer(64) | This value is increased in case of any change in the order book. Please note: revision numbers of order book are stored in memory only (not persistent) on CS OTE system. After a restart of CS OTE system, the revision numbers of order books will start from 0 again. OTE system checks gap detection (missing number in the XBID revision sequence) for every order book of contract separately. If a gap is detected (e.g. due to network interruption), the OTE system will perform an initialization process for the order book and the revision number is reset to 0. The initialization process is performed only for the order book with the detected gap. |
|  | contract |  | FIELD | m |  | String | Contract code identifier (long name). |
|  | delivery_area_id |  | FIELD | m |  | String | Delivery Area to which the attached order books refer to. |
|  | last_price |  | FIELD | o |  | Integer(64) | Last traded price. |
|  | price_direction |  | FIELD | o |  | Integer | Defines the direction of the price movement with regard to the last 2 trades happened and that are relevant for this orderbook. Valid values are: -1: Price decreased 0: Price unchanged 1: Price increased |
|  | last_quantity |  | FIELD | o |  | Integer | Last traded quantity. |
|  | total_quantity |  | FIELD | o |  | Integer(64) | The total quantity traded during this trading session. |
|  | last_trade_time |  | FIELD | o |  | Timestamp | Timestamp of the last execution. |
|  | high_price |  | FIELD | o |  | Integer(64) | Highest traded price since the start of the trading period. |
|  | low_price |  | FIELD | o |  | Integer(64) | Lowest traded price since the start of the trading period. |
|  | sell_orders |  | FIELD | o | 0..n | Structure |  |
|  |  | order_id | FIELD | m |  | Integer(64) | Order Id as determined by the CS OTE system. |
|  |  | quantity | FIELD | m |  | Integer(32) | The quantity of the order which is exposed in that delivery area. |
|  |  | price | FIELD | m |  | Integer(64) | Limit price of the order in currency defined by contract. Value is multiplied by 100, e.g. 1 Euro = 100. |
|  |  | order_entry_time | FIELD | m |  | Timestamp | Timestamp of the order. |
|  |  | order_execution_restriction | FIELD | o |  | Enum | Execution restriction of the order. This field is set only in case of AON orders. “ORDER_EXECUTION_RESTRICTION_TYPE_AON”: AON Order. |
|  | buy_orders |  | FIELD | 0 | 0..n | Structure |  |
|  |  | order_id | FIELD | m |  | Integer(64) | Order Id as determined by the CS OTE system. |
|  |  | quantity | FIELD | m |  | Integer(32) | The quantity of the order which is exposed in that delivery area. |
|  |  | price | FIELD | m |  | Integer(64) | Limit price of the order in currency defined by contract. Value is multiplied by 100, e.g. 1 Euro = 100. |
|  |  | order_entry_time | FIELD | m |  | Timestamp | Timestamp of the order. |
|  |  | order_execution_restriction | FIELD | o |  | Enum | Execution restriction of the order. This field is set only in case of AON orders. “ORDER_EXECUTION_RESTRICTION_TYPE_AON”: AON Order. |

Table 18 - Public order books report message structure

#### Public Order Books Delta Report (PublicOrderBooksDeltaRprt)

| PublicOrderBooksDeltaRprt |  |
| --- | --- |
| Type: | Broadcast |
| Response to: | n/a |
| Broadcast: | Yes |
| Broadcast Routing Keys: | <product_name>.<delivery_area> |
| Roles: | <All> |

The PublicOrderBooksDeltaRprt message is sent when an active bid is implemented or modified. This message includes all bids that have been modified since the previous distribution of the PublicOrderBooksDeltaRprt for the specified contract.

The message format is the identical to that of the PublicOrderBooksResp message.

#### Message Request (MessageReq)

| MessageReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | <ALL> |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 2/10 |

A request for trading system messages, that were created in the trading system in the past. It is possible to request up to one day prior.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| MessageReq | MSG |  | 1 | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| type | FIELD | m |  | Enum | Defines what kinds of messages are returned, allowing filtering the messages on a request level. Valid Values: "MESSAGE_TYPE_ALL": Return all messages. "MESSAGE_TYPE_PUBLIC": Return only public messages. "MESSAGE_TYPE_PRIVATE": Return only private messages. |
| end_date | FIELD | m |  | Timestamp | Timestamp defining to which point in time the messages should be retrieved. |
| start_date | FIELD | m |  | Timestamp | Timestamp defining from which point in time the messages should be retrieved. It is possible only to retrieve messages from the last 1 day. |

Table 19 - Message request message structure

#### Message Report (MessageRprt)

| MessageRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | MsgReq (sent to the user-generated private response queue or a broadcast to market.broadcastQueue.<login-id>) |
| Broadcasted: | Yes |
| Broadcast Routing Keys: | PRTC_<partic_id> <product_name> <product_name>.PRTC_<partic_id> public |
| Roles: | <All> |

Messages from the trading system are sent in response to the MessageReq and subsequently distributed whenever a new message is created in the trading system.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| MessageRprt |  | MSG | m | 1 | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| messages |  | FIELD | o | 0..n | Structure |  |
|  | message_id | FIELD | m |  | Integer(64) | The message Id as assigned by the CS OTE system. |
|  | type | FIELD | m |  | Enum | Defines the message type. Valid Values: "MESSAGE_TYPE_PUBLIC": The message is a public message. "MESSAGE_TYPE_PRIVATE": The message is a private message. |
|  | contract | FIELD | o |  | String | Related underlying contract (if any). |
|  | message_code | FIELD | o |  | Integer | Message code of the message |
|  | timestamp | FIELD | m |  | Timestamp | Timestamp of the message as assigned by the CS OTE system. |
|  | severity | FIELD | m |  | Enum | Severity of the message: "MESSAGE_SEVERITY_TYPE_URG": Urgent message. "MESSAGE_SEVERITY_TYPE_ERR": Error. "MESSAGE_SEVERITY_TYPE_HIG": High prioritized message. "MESSAGE_SEVERITY_TYPE_MED": Medium prioritized message. "MESSAGE_SEVERITY_TYPE_LOW": Low priority message. |
|  | market_supervision_message | FIELD | m |  | Boolean | Determines if the message has been send by market supervision |
|  | text_en | FIELD | m |  | String | Message text. – English version. |
|  | text_cz | FIELD | m |  | String | Message text – Czech version. |
|  | sell_delivery_area_id | FIELD | o |  | String | In case of an order execution, this field contains the delivery area of the sell side. |
|  | buy_delivery_area_id | FIELD | o |  | String | In case of an order execution, this field contains the delivery area of the buy side. |

Table 20 - Message report message structure

#### Trade Capture Request (TradeCaptureReq)

| TradeCaptureReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 7/35 |

A custom trades request. It is possible to request data up to seven days prior with a maximum date range of 24 hours. If the input parameters are invalid, the ErrResp is returned.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| TradeCaptureReq | MSG |  |  |  |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| start_date | FIELD | m |  | Timestamp | Start of the period for which the trades are retrieved. This value must fulfil the following conditions: - end_date – start_date <= 24 hours |
| end_date | FIELD | o |  | Timestamp | End of the period for which the trades are retrieved. The following condition must be fulfilled: - end_date – start_date <= 24 hours If no end_date is given, the CS OTE system will return all trades until midnight of the start date. In case of invalid value Error Message is returned stating that difference is bigger than max value. |

Table 21 - Trade capture request message structure

#### Trade Capture Report (TradeCaptureRprt)

| TradeCaptureRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | TradeCaptureReq (sent to the user-generated private response queue or a broadcast to market. broadcastQueue.<login-id>) |
| Broadcasted: | Yes |
| Broadcast Routing Keys: | halftrade.<product_name>.PRTC_<partic_id> |
| Roles: | EmtasImTsAcc |

A message that contains information about a trade creation or modification. It is sent to both participants of the specified trade (or only to one in the case of an international trade). For each receipient only the relevant part of the trade is included. This message is also sent in response to TradeCaptureReq and TradeRecallReq.

This message is also distributed in the following cases:

- A request for a cross-border CZ trade callback initiated from a different country, which modifies the trade status,

- A trade cancellation initiated by the central XBID side.

| Message/Field |  |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TradeCaptureRprt |  |  | MSG | m | 1 | Structure |  |
| standard_header |  |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| trades |  |  | FIELD | o | 0..n | Structure |  |
|  | trade_id |  | FIELD | m |  | Integer(64) | Trade ID of the trade. |
|  | revision_no |  | FIELD | m |  | Integer(64) | Revision number of this trade. With every change of the trade the revision number is increased by one. |
|  | state |  | FIELD | m |  | Enum | Current state of the trade. Valid values are: "TRADE_STATE_TYPE_ACTI": Trade is active (this is the default value). "TRADE_STATE_TYPE_CNCL": Trade was cancelled. "TRADE_STATE_TYPE_RREQ": Recall of this trade was requested. "TRADE_STATE_TYPE_RREJ": Recall request was rejected - trade is still valid then. "TRADE_STATE_TYPE_RGRA": Recall request was granted – trade has been recalled. |
|  | contract |  | FIELD | m |  | String | Contract code (long name). |
|  | quantity |  | FIELD | m |  | Integer | Executed quantity. |
|  | price |  | FIELD | m |  | Integer(64) | Execution price in currency defined by contract. Value is multiplied by 100, e.g. 1 Euro = 100. |
|  | execution_time |  | FIELD | m |  | Timestamp | Execution date as assigned by system. |
|  | latest_recall_process_time |  | FIELD | o |  | Timestamp | Informs until when a recall request can be processed. |
|  | recall_req_time |  | FIELD | o |  | Timestamp | Date and time of a recall request. |
|  | recall_granted_time |  | FIELD | o |  | Timestamp | Date and time when the recall was granted or when the trade was cancelled. |
|  | recall_rejected_time |  | FIELD | o |  | Timestamp | Date and time when the recall was rejected. |
|  | contract_phase |  | FIELD | m |  | Enum | “CONTRACT_PHASE_TYPE_CONT”: The trading in the contract is in continuous mode. "CONTRACT_PHASE_TYPE_AUCT": The trade results from an Intraday Micro Auction. |
|  | buy |  | FIELD | o | 0..1 | Structure |  |
|  |  | order_id | FIELD | m |  | Integer(64) | Order Id of the buy side order. |
|  |  | delivery_area_id | FIELD | m |  | String | Delivery Area to which the attached order books refer to. |
|  |  | partic_id | FIELD | m |  | Integer(64) | Participant who entered the buy side order. |
|  |  | user_id | FIELD | m |  | Integer(64) | User id of the user who entered the buy side order. |
|  |  | client_order_id | FIELD | o |  | String | Client’s identification of order. |
|  |  | text | FIELD | o |  | String | Text of the buy side order. |
|  |  | initiator_or_aggressor | FIELD | o |  | Enum | “INITIATOR_AGGRESSOR_TYPE_I”: the buy order was already in the market before it was matched “INITIATOR_AGGRESSOR_TYPE_A”: the buy order was matched at the moment it was introduced in the market “INITIATOR_AGGRESSOR_TYPE_N”: the trade comes out of an auction |
|  | sell |  | FIELD | o | 0..1 | Structure |  |
|  |  | order_id | FIELD | m |  | Integer(64) | Order Id of the sell side order. |
|  |  | delivery_area_id | FIELD | m |  | String | Delivery Area to which the attached order books refer to. |
|  |  | partic_id | FIELD | m |  | Integer(64) | Participant who entered the sell side order. |
|  |  | user_id | FIELD | m |  | Integer(64) | User id of the user who entered the sell side order. |
|  |  | client_order_id | FIELD | o |  | String | Client’s identification of order. |
|  |  | text | FIELD | o |  | String | Text of the sell side order. |
|  |  | initiator_or_aggressor | FIELD | o |  | Enum | “INITIATOR_AGGRESSOR_TYPE_I”: the sell order was already in the market before it was matched “INITIATOR_AGGRESSOR_TYPE_A”: the sell order was matched at the moment it was introduced in the market “INITIATOR_AGGRESSOR_TYPE_N”: the trade comes out of an auction |

Table 22 - Trade capture report message structure

#### Public Trade Confirmation Request (PublicTradeConfirmationReq)

| PublicTradeConfirmationReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 7/35 |

A request for public information about established trades. It is possible to request data up to 7 days prior with a maximum date range of 24 hoursIf the input parameters are invalid, the ErrResp is returned.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| PublicTradeConfirmationReq | MSG | m |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| start_date | FIELD | m |  | Timestamp | Start of the period for which the trades are retrieved. This value must fulfil the following conditions: - end_date – start_date <= 24 hours |
| end_date | FIELD | o |  | Timestamp | End of the period for which the trades are retrieved. The following condition must be fulfilled: - end_date – start_date <= 24 hours If no end_date is given, the system will return all trades until midnight of the start_date. In case of invalid value Error Message is returned stating that difference is bigger than max value. |
| product_names | FIELD | o | 0.. 1000 | String | List of product names for which the public trade confirmations are requested. If not supplied all products for which the user has access rights are returned |

Table 23 - Public trade confirmation request message structure

#### Public Trade Confirmation Report (PublicTradeConfirmationRprt)

| PublicTradeConfirmationRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | PublicTradeConfirmationReq (sent to the user-generated private response queue or a broadcast to market. broadcastQueue.<login-id>) |
| Broadcasted: | Yes |
| Broadcast Routing Keys: | public.trade.<product_name> |
| Roles: | EmtasImTsAcc |

A message containing a trade establishment. This message is distributed to all users assigned to the contract relevant to the established trade. It is also sent in response to PublicTradeConfirmationReq.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| PublicTradeConfirmationRprt |  | MSG | m | 1 | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| trades |  | FIELD | o | 0..n | Structure |  |
|  | trade_id | FIELD | m |  | Integer(64) | Trade Id of the underlying trade. |
|  | revision_no | FIELD | m |  | Integer(64) | Revision number of the trade. This is increased by one every time the trade is changed. |
|  | state | FIELD | m |  | Enum | Current state of the trade. Valid values are: "TRADE_STATE_TYPE_ACTI": Trade is active (this is the default value). "TRADE_STATE_TYPE_CNCL": Trade was cancelled. "TRADE_STATE_TYPE_RREJ": Recall request was rejected - trade is tsill valid. "TRADE_STATE_TYPE_RGRA": Recall request was granted – trade has been recalled. "TRADE_STATE_TYPE_RREQ": Recall of this trade was requested. |
|  | contract | FIELD | m |  | String | Contract code (long name) of the trade. |
|  | price | FIELD | m |  | Integer(64) | Execution price in currency defined by contract. Value is multiplied by 100, e.g. 1 Euro = 100. |
|  | quantity | FIELD | m |  | Integer | Traded quantity. |
|  | trade_execution_time | FIELD | m |  | Timestamp | Trade execution time. |
|  | sell_delivery_area_id | FIELD | o |  | String | Delivery Area of the sell part. |
|  | buy_delivery_area_id | FIELD | o |  | String | Delivery Area of the buy part. |

Table 24 - Public trade confirmation report message structure

#### Contract Information Request (ContractInfoReq)

| ContractInfoReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | <ALL> |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 10/40 |

A contract request. It is possible to request data up to 7 days prior. If the input parameters are invalid, ErrResp is returned in the response.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| ContractInfoReq | MSG |  |  | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| start_date | FIELD | o |  | Timestamp | Start date for which the contract information is requested. Notes: - if contract field is specified this field is ignored - if product_names field is specified or neither contract nor product_names fields are specified, this field becomes mandatory. |
| end_date | FIELD | o |  | Timestamp | End date for which the contract information is requested. Notes: - if contract field is specified this field is ignored - if product_names field is specified or neither contract nor product_names fields are specified, this field becomes mandatory. |
| product_names | FIELD | o | 0.. 1000 | String | The contract information for all contratcs belonging to products with given product names is requested. If product_names field is specified, the contract field cannot be specified and the start_date and end_date fields are mandatory. |
| contract | FIELD | o | 0..1 | String | Contract code (long name). If contract is specified, the products field cannot be specified and the start_date and end_date fields are ignored. |

Table 25 - Contract information request message structure

#### Contract Information Report (ContractInfoRprt)

| ContractInfoRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | ContractInfoReq (sent to the user-generated private response queue or a broadcast to market. broadcastQueue.<login-id>) |
| Broadcasted: | Yes |
| Routing Keys: | <product_name> |
| Roles: | EmtasImTsAcc |

A contract information. This message is distributed whenever any contract attribute is modified or in response to the ContractInfoReq request.

| Message/Field |  |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ContractInfoRprt |  |  | MSG | m | 1 | Structure |  |
| standard_header |  |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
|  | contracts |  | FIELD | o | 0..n |  |  |
|  | contract_id |  | FIELD | m |  | Integer(64) | Contract ID. |
|  | revision_no |  | FIELD | m |  | Integer(64) | Revision number of the contract. |
|  | product_name |  | FIELD | m |  | String | Underlying product. |
|  | product_revision_no |  | FIELD | m |  | Integer(64) | Revision number of the underlying product. |
|  | name |  | FIELD | m |  | String | Contract name. This is used for display purposes. |
|  | long_name |  | FIELD | m |  | String | Contract long name, containing additional information. |
|  | delivery_start |  | FIELD | m |  | Timestamp | Start of delivery. |
|  | delivery_end |  | FIELD | m |  | Timestamp | End of delivery. |
|  | duration |  | FIELD | o |  | Double | The duration of the contract in full hours. For quarterly contracts the value would be 0.25. An hourly contract would have 1.0. An block contract would have value in interval from 2 to 24 (or 23/25 in case of short/long clock change). |
|  | predefined |  | FIELD | m |  | Boolean | Flag that indicates, if a contract has been automatically created by the system or if the contract was generated with an entry of a user-defined block order. 1 = automatically generated, 0= user defined |
|  | state |  | FIELD | m |  | Enum | Current state of the contract. The following values are allowed: "CONTRACT_STATE_TYPE_HIBE": Hibernated, the contract was manually deactivated by Central Admin. "CONTRACT_STATE_TYPE_ISSUED": The contract is issued, but not available for trading. "CONTRACT_STATE_TYPE_OPEN": Contract is active and available for trading. "CONTRACT_STATE_TYPE_CLOSE": Contract is closed and not available for trading. "CONTRACT_STATE_TYPE_TERM": Contract is terminated and not available for trading. "CONTRACT_STATE_TYPE_NOT_ISSD": The contract is not issued. |
|  | trading_phase_start |  | FIELD | o |  | Timestamp | Start date and time of the current/next trading phase. |
|  | trading_phase_end |  | FIELD | o |  | Timestamp | End date and time of the current/next trading phase. |
|  | delivery_area_states |  | FIELD | o | 0..n | Structure |  |
|  |  | delivery_area_id | FIELD | m |  | String | Delivery Area identification |
|  |  | trading_phase_start | FIELD | m |  | Timestamp | Start date and time of the current/next trading phase. |
|  |  | trading_phase_end | FIELD | o |  | Timestamp | End date and time of the current/next trading phase. |
|  |  | state | FIELD | m |  | Enum | Current state of the delivery area. The following values are allowed: "AREA_STATE_TYPE_IACT": Delivery area is inactive and thus not tradable. "AREA_STATE_TYPE_ACTI": Delivery area is active. It is possible to trade in that area. "AREA_STATE_TYPE_HIBE": Delivery area is deactivated (hibernated). Trading in that delivery area is not possible. |
|  |  | trading_phase | FIELD | m |  | Enum | “CONTRACT_PHASE_TYPE_CONT”: The contract is in continuous mode. "CONTRACT_PHASE_TYPE_AUCT": The contract is in Micro Auction mode. "CONTRACT_PHASE_TYPE_CLSD": The contract is in Close phase. |

Table 26 - Contract information report message structure

#### Product Information Request (ProductInfoReq)

| ProductInfoReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 2/20 |

A detailed product information request.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| ProductInfoReq | MSG | m | 1 | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| product_names | FIELD | o | 0.. 1000 | String | List of product names. |

Table 27 - Product information request message structure

#### Product Information Report (ProductInfoRprt)

| ProductInfoRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | ProdInfoReq (sent to the user-generated private response queue or a broadcast to market. broadcastQueue.<login-id>) |
| Broadcasted: | Yes |
| Broadcast Routing Keys: | <product_name> |
| Roles: | EmtasImTsAcc |

A detailed product information as a response to the ProductInfoReq.

Since ContractInfoReq allows requests up to 7 days prior (see also 2.8.4.10 Contract Information Request (ContractInfoReq)), this response returns only product revisions that even the latest contract can reference.

The response may also include multiple product versions with the same name, where the unique identifier is the product name combined with the revision number.

| Message/Field |  |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ProductInfoRprt |  |  | MSG | m | 1 | Structure |  |
| standard_header |  |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| products |  |  | FIELD | o | 0..n | Structure |  |
|  | product_name |  | FIELD | m |  | String | Unique identifier name of the product. |
|  | display_name |  | FIELD | m |  | String | String used to display the product. |
|  | currency |  | FIELD | m |  | String | The currency of the product. The value is always “EUR”. |
|  | revision_no |  | FIELD | m |  | Integer(64) | Revision number of the product. This value is increased by one every time the product is modified by the system. |
|  | quantity_unit |  | FIELD | m |  | String | Defines the quantity unit. |
|  | min_quantity |  | FIELD | o |  | Integer | Minimal display quantity. |
|  | decimal_shift_quantity |  | FIELD | m |  | Integer | Decimal shift of the quantity information. A value of 2 results in a display of 100 kW. |
|  | max_quantity |  | FIELD | m |  | Integer | Maximal allowed quantity for orders entered in contracts belonging to this product. |
|  | min_price |  | FIELD | m |  | Integer(64) | Minimal price allowed for orders entered in contracts belonging to this product. |
|  | max_price |  | FIELD | m |  | Integer(64) | Maximal price allowed for orders entered in contracts belonging to this product. |
|  | decimal_shift_price |  | FIELD | m |  | Integer | Decimal shift of the price information. A value of 2 results in a display in Eurocents. |
|  | contract_name_pattern |  | FIELD | o |  | String | Format string for the contract name. |
|  | tick_size |  | FIELD | m |  | Integer | Defines the minimum increment for limit prices for this product. The value is entered as an integer, but the decimal price shift is applied. |
|  | lot_size |  | FIELD | m |  | Integer | Defines the smallest tradable unit of the product. |
|  | product_configurations |  | FIELD | o | 0..n | Structure |  |
|  |  | key | FIELD | m |  | String | Exchange specific product attribute names (e.g.: blockOrderProduct, icebergMinPeakSize, icebergPriceDeltaRange) |
|  |  | value | FIELD | m |  | String | Exchange specific product attribute values |

Table 28 - Product information report message structure

#### Market State Request (MarketStateReq)

| MarketStateReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 2/20 |

A current market status request. The required market is specified in the message header standard_header.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| MarketStateReq | MSG | m | 1 | Structure |  |
| standard_header | FIELD |  |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |

Table 29 - Market state request message structure

#### Market State Report (MarketStateRprt)

| MarketStateRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | MktStateReq (sent to the user-genereted private response queue or a broadcast to market.broadcastQueue.<login-id>) |
| Broadcasted: | Yes |
| Broadcast Routing Keys: | public.<market_id> |
| Roles: | EmtasImTsAcc |

Current information about the market trading status. This message is distributed whenever the market status is modified and in response to the MarketStateReq request.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| MarketStateRprt | MSG | m | 1 | Structure |  |
| standard_header | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| state | FIELD | m |  | Enum | Contains the current market state. The following values are allowed: "MARKET_STATE_TYPE_HIBE": Hibernated; no trading is possible and order books are empty. Done on WebGui by Admin. "MARKET_STATE_TYPE_ACTI": Market is active and trading is possible. |
| connected_xbid | FIELD | o |  | Enum | State identification of physical connection to XBID solution. “CONNECTED_XBID_TYPE_ACTI” – Connection to XBID solution is valid. “CONNECTED_XBID_TYPE_DISC” – Disconnected from XBID solution. Used only for market_id “MARKET_ID_TYPE_XBID”. |
| trading_xbid | FIELD | o |  | Enum | “TRADING_XBID_TYPE_OPER” – Trading on XBID is allowed by OTE at OTE-COM (in operation). “TRADING_XBID_TYPE_SUSP” – Trading on XBID is suspended by OTE at OTE-COM. Used only for market_id “MARKET_ID_TYPE_XBID”. |
| revision_no | FIELD | m |  | Integer(64) | Revision number of the market. With every change of the market state this value is increased by one. |

Table 30 - Market state report message structure

#### Hub-to-Hub ATC Matrix Request (HubToHubReq)

| HubToHubReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 2/10 |

This request provides Hub-to-Hub data capacity matrix acquisition.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| HubToHubReq | MSG | m |  | Structure |  |
| standard_header | FIELD |  |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| delivery_area | FIELD | m |  | String | Delivery Area |
| delivery_day | FIELD | m |  | Timestamp | Date |

Table 31 - Hub-to-Hub matrix request message structure

#### Hub-to-Hub Matrix Report (HubToHubResp)

| HubToHubResp |  |
| --- | --- |
| Type: | Inquiry Response |
| Response to: | HubToHubReq (sent to private autogenerated response queue) |
| Broadcast: | No |
| Roles: | EmtasImTsAcc |

This message is sent in response to HubToHubReq. It is sent to the private message queue of the user, who submitted the corresponding HubToHubReq request.

| Message/Field |  |  |  |  | Type |  | m/o |  | No. |  | Data Type |  | Short description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HubToHubResp |  |  |  |  | MSG |  | m |  |  |  | Structure |  |  |  |
| standard_header |  |  |  |  | FIELD |  | m |  |  |  | Structure |  | Standard header of each message. Please see chapter 2.6.7 Standard message header. |  |
| hub_to_hub_atcs |  |  |  |  | FIELD |  | m |  | 1..n |  | Structure |  |  |  |
|  | delivery_start |  |  | FIELD |  | m |  |  |  | Timestamp |  | Delivery start date |  |
|  | delivery_end |  |  | FIELD |  | m |  |  |  | Timestamp |  | Delivery end date |  |
|  | timestamp |  |  | FIELD |  | m |  |  |  | Timestamp |  | Timestamp when the ATC data was received from the Capacity system |  |
|  |  | hub_froms |  | FIELD |  | o |  | 0..n |  | Structure |  |  |  |
|  |  | from |  | FIELD |  | m |  |  |  | String |  | The outgoing Delivery area code |  |
|  |  | atcs |  | FIELD |  | o |  | 0..n |  | Structure |  |  |  |
|  |  |  | to | FIELD |  | m |  |  |  | String |  | Inbound Delivery area code |  |
|  |  |  | in | FIELD |  | m |  |  |  | Integer |  | Available capacity DA(to)-DA(from) |  |
|  |  |  | out | FIELD |  | m |  |  |  | Integer |  | Available capacity DA(from)-DA(to) |  |

Table 32 - Hub-to-Hub matrix report message structure

#### Hub-to-Hub Notification (HubToHubNtfRprt)

| HubToHubResp |  |
| --- | --- |
| Type: | Broadcast |
| Broadcast: | Yes |
| Broadcast Routing Keys: | public.<market_id> |
| Roles: | EmtasImTsAcc |

This message is distributed automatically whenever the Hub-to_Hub data matrix is modified (for example when a cross-border trade is established or in the case of an explicit allocation).

| Message/Field |  |  |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HubToHubNtfRprt |  |  |  | MSG | m |  | Structure |  |
| standard_header |  |  |  | FIELD |  |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| hub_to_hub_atcs |  |  |  | FIELD | m | 1..n | Structure |  |
|  | delivery_start |  |  | FIELD | m |  | DateTime | Delivery start date |
|  | delivery_end |  |  | FIELD | m |  | DateTime | Delivery end date |
|  | timestamp |  |  | FIELD | m |  | DateTime | Timestamp when the ATC data was received from the Capacity system |
|  | hub_froms |  |  | FIELD | o | 0..n | Structure |  |
|  |  | from |  | FIELD | m |  | String | The outgoing Delivery area code |
|  |  | atcs |  | FIELD | o | 0..n | Structure |  |
|  |  |  | to | FIELD | m |  | String | Inbound Delivery area code |
|  |  |  | in | FIELD | m |  | Integer | Available capacity DA(to)-DA(from) |
|  |  |  | out | FIELD | m |  | Integer | Available capacity DA(from)-DA(to) |

Table 33 - Hub-to-Hub matrix report message structure

### IM reference data

This chapter describes the message structure including information about the delivery and market area. Since the central XBID side distributes data only for the Czech market and delivery area to OTE, MP OTE also receive information only for the Czech area.

#### Delivery Area Information Request (DeliveryAreaInfoReq)

| DeliveryAreaInfoReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 1/10 |

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| DeliveryAreaInfoReq | MSG | m |  | Structure |  |
| standard_header | FIELD |  |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| product_names | FIELD | o | 0.. 1000 | String | List of products. |

Table 34 - Delivery are information request message structure

#### Delivery Area Information Report (DeliveryAreaInfoRprt)

| DeliveryAreaInfoRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | DeliveryAreaInfoReq (sent to private autogenerated response queue) |
| Broadcast: | Yes |
| Broadcast Routing Keys: | public.<market_id> |
| Roles: | EmtasImTsAcc |

This message is distributed whenever a delivery area attribute is modified. It is also sent in response to the DeliveryAreaInfoReq request.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| DeliveryAreaInfoRprt |  | MSG | m | 1 | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| delivery_areas |  | FIELD | o | 0..n | Structure |  |
|  | delivery_area_id | FIELD | m |  | String | Delivery Area ID. |
|  | revision_no | FIELD | m |  | Integer(64) | Revision number. With every change of the delivery area this value is increased by one. |
|  | name | FIELD | m |  | String | Name of the delivery area usually used for display purposes. |
|  | long_name | FIELD | m |  | String | Long name of the delivery area. |
|  | state | FIELD | m |  | Enum | Current state of the delivery area. The following values are allowed: "AREA_STATE_TYPE_IACT": Delivery area is inactive and thus not tradable. "AREA_STATE_TYPE_ACTI": Delivery area is active. It is possible to trade in that area. "AREA_STATE_TYPE_HIBE": Delivery area is deactivated (hibernated). Trading in that delivery area is not possible. |
|  | market_area_id | FIELD | m |  | String | ID of the Market Area this delivery area belongs to. |
|  | product_names | FIELD | o | 0..n | String | List of assigned products. In case of a state change for a delivery area, this list is not provided. |

Table 3536 - Delivery area information report message structure

#### Market Area Information Request (MarketAreaInfoReq)

| MarketAreaInfoReq |  |
| --- | --- |
| Type: | Inquiry Request |
| Roles: | EmtasImTsAcc |
| Routing Keys: | market.request.inquiry |
| Request Limits: | 1/10 |

This message provides acquisition of market area information.

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| MarketAreaInfoReq | MSG | m |  | Structure |  |
| standard_header | FIELD |  |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| product_names | FIELD | o | 0.. 1000 | String | List of products. |

Table 37 - Market area information request message structure

#### Market Area Information Report (MarketAreaInfoRprt)

| MarketAreaInfoRprt |  |
| --- | --- |
| Type: | Inquiry Response, Broadcast |
| Response to: | MarketAreaInfoReq (sent to private autogenerated response queue) |
| Broadcast: | Yes |
| Broadcast Routing Keys: | public.<market_id> |
| Roles: | EmtasImTsAcc |

This message is distributed whenever a market area attribute is modified. It is sent in response to the MarketAreaInfoReq request.

| Message/Field |  | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- | --- |
| MarketAreaInfoRprt |  | MSG | m | 1 | Structure |  |
| standard_header |  | FIELD | m |  | Structure | Standard header of each message. Please see chapter 2.6.7 Standard message header. |
| market_areas |  | FIELD | o | 0..n | Structure |  |
|  | market_area_id | FIELD | m |  | String | Market Area ID. |
|  | name | FIELD | m |  | String | Name of the market area usually used for display purposes. |
|  | long_name | FIELD | m |  | String | Long name of the market area usually. |
|  | state | FIELD | m |  | Enum | Current state of the market area. The following values are allowed: "AREA_STATE_TYPE_IACT": Market area is inactive and thus not tradable. "AREA_STATE_TYPE_ACTI": Market area is active. It is possible to trade in that area. "AREA_STATE_TYPE_HIBE": Market area is deactivated (hibernated). Trading in that market area is not possible. |
|  | revision_no | FIELD | m |  | Integer(64) | Revision number. With every change of the market area this value is increased by one. |

Table 38 - Market area information report message structure

## Scenarios for the current automatic communication through the KSP/KSM communication server

### Configuration/modification/response to the new IM limit

The current IM limit state including additional values, which returns a modified current limit state in the form of an established SFVOTLIMITS structure.

The structure SFVOTSETTINGS provides an IM limit modification through AC(KSP). Apart from the standard header and the receipient identification it also contains:

SFVOTSETTINGS/Setting –main encapsulating data element

SFVOTSETTINGS/Limit –main element used for limit configuration

SFVOTSETTINGS/Limit@type – limit, enum type, currently in IM only

SFVOTSETTINGS/Limit@value – new value for the specified limit in CZK

An example where 20 000 CZK is set up as limit:

<?xml version="1.0" encoding="UTF-8" standalone="yes"?>

<SFVOTSETTINGS answer-required="false" date-time="2015-06-24T12:41:08+02:00" dtd-release="1" dtd-version="1" id="123" message-code="481" xmlns="http://www.ote-cr.cz/schema/sfvot/settings">

<SenderIdentification id="8591824000007" coding-scheme="14"/>

<ReceiverIdentification id="8591824000007" coding-scheme="14"/>

<Setting>

<Limit type="VDT" value="20000"/>

</Setting>

</SFVOTSETTINGS>

The response contains the RESPONSE structure with msg code 483 and in case of a successful run also a data copy (where SFVOTLIMITS contains msg code 482). The current return codes from financial reporting area are used.

| RESPONSE/Reason@code | Popis |
| --- | --- |
| S09000 | The request has been processed successfully, the configuration has been changed. |
| S09008 | The participant does not have the required configuration (the limits are undefined). |
| S09009 | The participant is not authorized to make the modification. |
| S09010 | Insufficient funds. |
| S09011 | Invalid value. |
| S09012 | Unexpected error. |

Table 39 - Response reason codes with msg code 483

### Message indicating the transfer of a portion of the IM limit into the main trading limit

During the partial transfer of the IM limit into the main trading limit, which occurs automatically when establishing a trade that depletes the funds of the main trading limit, it is also necessary to notify the participant through the AC. Information sent to the participant is the following:

- Transferred financial value from the IM limit to the main trading limit (CZK)

- Remaining value of the IM limit (CZK)

- Remaining available financial funds in the IM settlement (CZK)

- ID of the trade that initiated the transfer

- Trade delivery date

The SFVOTLIMITCHANGE structure is used for this purpose. It is sent unsolicited via the KSP. Apart from the standard header and the receipient and sender identification, it also contains:

SFVOTLIMITCHANGE/Limits – main encapsulating data element

SFVOTLIMITCHANGE/Limits@trade-date – trade delivery date

SFVOTLIMITCHANGE/Limits@trade-id – trade id

SFVOTLIMITCHANGE/Limit – main limit element

SFVOTLIMITCHANGE/Limit@type – limit type, enum type, currently only IM

SFVOTLIMITCHANGE/Limit@value – new value of the specific limit in CZK

SFVOTLIMITCHANGE/Limit@moved – funds transferred into a different type in CZK (for IM to utilization of short-term trades in the main trading limit)

SFVOTLIMITCHANGE/Limit@free – funds available for the specific limit in CZK

Example:

<?xml version="1.0" encoding="UTF-8" standalone="yes"?>

<SFVOTLIMITCHANGE answer-required="false" date-time="2015-06-24T12:41:08+02:00" dtd-release="1" dtd-version="1" id="123" message-code="484 xmlns="http://www.ote-cr.cz/schema/sfvot/limitchange">

<SenderIdentification id="8591824000007" coding-scheme="14"/>

<ReceiverIdentification id="8591824000007" coding-scheme="14"/>

<Limits trade-id="237445" trade-date="2015-08-31">

<Limit type="VDT" value="15000" moved="5000" free="1280"/>

</Limits>

</SFVOTLIMITCHANGE>

# Using the electronic signature

Messages are transferred between the client application and the backend system in a binary protobuf format. To ensure integrity and indisputability, specific messages are secured via the electronic signature.

Electronic signature security includes the following messages:

- ModifyOrderReq

- AddOrderReq

- ModifyAllOrdersReq

The structures of the above mentioned messages will, after the introduction of the electronic signature, become part of the SignedMessage structure, which will contain an item:

- Content type bytes, which is the original message and the digital signature in a binary CMS format, serialized into a byte field before the signature is created

Figure 19 - SignedMessage creation

After the signature and certificate validation on the receipient’s side, the original message must be extracted from the CMS format and based on the message type, deserialized into the relevant objects for further processing.

Figure 20 - Digital signature message verification with original message extraction

For a digital signature, it is necessary to use the standard CMS defined in RFC 5625. It is a message of signed-data type, which contains the SignedData ASN.1 structure. It contains the original message, the signature and the certificate corresponding to the private key used for signing. For the hash function, at least the SHA256 algorithm or a stronger one must be used.

SignedMessage structure definition:

| Message/Field | Type | m/o | No. | Data Type | Short description |
| --- | --- | --- | --- | --- | --- |
| SignedMessage | MSG |  |  | Structure |  |
| content | FIELD | m |  | Bytes | Original binary message together with digital signature in binary format. |
| messageType | FIELD | m |  | String | Contains name of the signed message. |
| contentEncoding | FIELD | o |  | String | Contains gzip, if the message was compressed prior to being signed. |

Table 40 - SignedMessage message structure

The expected interval is 5 seconds, however it will be specified later during the project implementation.