# Use of certificates in Automatic Communication since 23rd August

OTE use 3 certificates and their root authorities for Automatic Communication:

1) CS OTE - it is issued by a qualified authority. It can be used only for electronic signature. Transparent TLS authentication is not possible by the qualified certificates.

2) CSOTE Secure MIME - encryption of communication from the Participant to OTE + newly OTE authentication when calling Participant´s system.

3) OTE OTECA server certificate - will be replaced later by certificate from another certification authority, but so far it remains the same.

Root Authorities

4) Commercial I.CA Intermediate Authority

5) Qualified I.CA Intermediate Authority

6) OTECA root authority

7) Root Authority I.CA

Their use will be as follows:

# Server-server communication

### Calling the service in the direction Participant-> OTE

TLS: Participant calls OTE, using either a commercial certificate of supported authority for authentication or does not use any certificate. CS OTE returns the OTECA certificate (public part)(3) to be used for communication encryption.

SOAP: Participant uses qualified certificate for the ws-security signature, eventually also to sign an XML message. Message returned from OTE will be signed by CS OTE (1) certificate.

### Calling the service in the direction OTE-> Participant

TLS: OTE calls the participant using the CS OTE Secure MIME certificate (3) for authentication, the participant returns his server certificate that will be used for encryption of communication.

SOAP: OTE uses the CS OTE certificate (1) for the signature ws-security, eventually to sign an XML message.

### Necessary changes:

Participants have to check if they have I.CA commercial authority certificates (4, 7) in the truststore for TLS client certificate authentication. If participant uses additional validation mechanism, it has to be adapted to new CS OTE Secure MIME (2) certificate, e.g. DN mapping to routing rules, certificate mapping (or just CN) to the system user.

I.CA qualified authority certificates (5, 7) must be added to the ws-security truststore. If participant uses additional validation mechanism, it has to be adapted to new CS OTE certificate (1).

OTECA authority (6) has to remain in the truststore because OTE server certificate (3) still will be used for encryption of the connection in the direction Participant-> OTE.

# Client-server communication

TLS: Participant calls OTE, using either a commercial certificate of authority supported for authentication, or does not need a certificate. CS OTE returns the OTECA certificate (3) to be used for communication encryption

SOAP: Participant uses qualified certificate for the ws-security signature, eventually also to sign an XML message. Message returned from OTE will be signed by CS OTE (1) certificate.

### Necessary changes:

I.CA qualified authority certificates (5, 7) must be added to the ws-security truststore. If participant uses additional validation mechanism, it has to be adapted to new CS OTE certificate (1).

OTECA authority (6) has to remain in the truststore because OTE server certificate (3) still will be used for encryption of the connection in the direction Participant-> OTE

# SMTP e-mail communication

### Direction participant->OTE

Participant signs the mail by his qualified certificate and encrypt it with the CS OTE secure MIME certificate (2)

### Direction OTE->participant

OTE signs e-mail by CS OTE (1) certificate and encrypt it with participant’s commercial certificate.

### Necessary changes:

Participants have to change CS OTE secure MIME (2) in their e-mail clients.Participants have to install I.CA root Authority in to the internet explorer (windows trustsotre) in to the root authorities and Commercial I.CA Intermediate Authority(4) and Qualified I.CA Intermediate Authority(5) in to the Intermediate Authorities.

Certificate references valid from 23.8.2017:

- CS OTE – http://www.ote-cr.cz/registrace-a-smlouvy/pristup-do-cs-ote/files-konfigurace-pc/csote-2017.cer

- CSOTE Secure MIME – http://www.ote-cr.cz/registrace-a-smlouvy/pristup-do-cs-ote/files-konfigurace-pc/csote-smime-2017.cer

- OTECA - OTE server certificate – no change

Root Authorities

- Commercial I.CA Intermediate Authority - http://www.ica.cz/CA-for-commercial-services

- Qualified I.CA Intermediate Authority - http://www.ica.cz/CA-for-qualified-services

- OTECA root authority - http://www.ote-cr.cz/registrace-a-smlouvy/pristup-do-cs-ote/files-konfigurace-pc/oteca_2010.cer

- Root Authority I.CA - http://www.ica.cz/HCA-root-en